FINITE ANTI-PLANE SHEAR DEFORMATION OF A NEO-HOOKEAN MATERIAL USING MONGE METHOD OF SOLUTION

Abstract

Finite deformation of an incompressible hollow  Neo-Hookean material under anti-plane shear is investigated. The problem is converted from Cartesian co-ordinate to cylindrical polar co-ordinate since the problem is better handled in cylindrical polar co-ordinate. The analysis produces an elliptic second order partial differential equation which sought for Monge Method of solution for the determination of displacement and stresses.  Boundary value conditions are set up in determining the contacts of integration involved in the solution. Finally a closed solution for the displacement and stresses at any cross section of the cylinder is achieved.

Keywords: Anti-plane shear, displacement,  deformed radius, shear stresses and monge mothed.

- Introduction

The major problem in the analysis of solids deforming under anti-plane shear is difficult to achieve closed form solutions to the boundary value problems resulting from the analysis. Anti-plane shear deformations are one of the simplest classes of deformations that solids can undergo. Anti-plane shear involve deformations of a cylindrical region in such a way that the displacement of each particle is parallel to the axial direction and independent of its axial coordinate. For the case of linear isotropic material like neo Hookean material, the strain energy function W =W(I1) where I1 is the first principal invariant of the left Cauchy Green Strain tensor

B=FFT, F being the gradient of deformation. The absent of I2 in the strain energy function enables us to achieve analytic solution with the boundary value problem. When we have linear isotropic elasticity, the three dimensional equations can be reduced to a single two dimensional second order equation which is the axial component of the equation for a single unknown usually denoted as the out of plane displacement. Erumaka (1) in one of his works investigated on “finite anti plane shear deformation of a class of Ogden containing a line crack”. Erumaka and Onugha (2) analyzed “the stresses and deformation in a Neo-Hookean Half space deforming under Anti plane shear loading”. Polygnone and Horgan (3) worked on “Axisymmetric finite Anti plane shear of compressible Non linearly elastic circular Tubes”. Ejike and Erumaka (4) investigated “finite deformation of a rotating circular cylinder of Blatz-ko material”. Simmand and Warne (5) studied “Azimuthal shear of compressible non-linaerly Elastic orthotropic Tubes of finite Extent”. Horgan and Giuseppe (6) analyzed  “Anti-plane shear deformations for non-Gaussian isotropic, incompressible hyperelastic materials”. Also Horgan and Giuseppe (7) extended “the work to Superposition of Generalized plane strain on anti-plane shear deformations in isotropic incompressible Hyperelastic”. Lejla (8) did work on “Two parametric analysis of Anti-plane shear deformation of a coated Elastic Half-Space” and Horgan (9) investigated on “Anti-plane shear deformations in linear and non linear solid mechanics”. Moreover Yang (10) investigated “Asymptotic solution to Axisymmetric indention of a compressible elastic thin film”. Darijani and Bahremen (11) used “polynomial hyperelastic models to obtain a closed form solution for analyses of rubbery solid circular cylinder”. Robert et al (12) with “the use of neo-Hookean and the Mooney–Rivlin models found the strain energy function for isotropic incompressible solids demonstrating a linear relationship between shear stress and amount of shear, and between torque and amount of twist, when subject to large simple shear or torsion deformations”. Merodio and Ogden (13) proposed “a new example of the solution to the finite deformation boundary value problem for a residually stressed elastic body and  combined extension, inflation and torsion of a circular cylindrical tube    subject to radial and circumferential residual stress. The isochoric deformation consisted of axial extension, radial inflation and superimposed torsion which is formulated for a general elastic strain energy function. Two simple strain energy functions incorporating radial stress were used and the integral were evaluated to give close form expressions for pressure, axial load and torsional movement. In addition to works done in elasticity on cylindrical materials”. Anani and Gholamhosein (14) worked on “spherical material, Stress analysis of thick pressure vessel composed of incompressible hyperelastic materials where Neo Hookean strain energy function was used to determine the stress and displacement of spherical shell that is axisymmetric radially deformed under internal and external pressure. Exact solutions were derived for stress and stretch in a thick hyperelastic spherical shell and the effect of the structure parameter for different examples was discussed”. Shearer et al (15) analyzed “Torsional wave propagation in a  pre-stressed hyperelastic annular circular cylinder”.

Grine et al (16) worked on Elastic machines, a non standard  use of the axial shear of linear transversely isotropic elastic cylinders. Marco and Giuseppe (17) investigated on superposing plane strain on anti-plane shear deformations in a special class of fibre-reforced incompressible hyperelastic materials.  .In this present paper we sought for analytic solution using Monge method of solution for the determination of stresses and displacement across a hollow cylinder made of Neo-hookean material.

- Governing Equations

Let the deformation of an isotropic, homogenous, incompressible, elastic material that takes the point (X1, X2, X3) of the initial configuration to the point (x1, x2, x3) of the deformed configuration in anti-plane shear be

x1=X1 , x2= X2 ,  x3=X3 +w(X1,X2)                                                (1)

Figure 1.: Anti-plane shear deformation

x3

x1

x2

w(X1,X2) is called out of plane displacement or anti-plane displacement and w depends continuously on  X1 and X2 coordinates and twice differentiable function of  X1 and X2 at every point of D and its deformed image D*.

The deformation gradient tensor F for equation (1) is given as

F=100010w1w21                                                                                     (2)

where    ∂w∂X1=w1    , ∂w∂X2=w2

The Left Cauchy-Green deformation gradient tensor B associated with (1) is given as

B=FFT=100010w1w2110w101w2001

B=10w101w2w1w21+w12+w22                                                              (3)

B-I=1+w12w1w2-w1w1w21+w12-w2-w1-w21

B2=B.B=1+w12w1w2w1(2+w12+w22)w1w21+w2w2(2+w12+w22)w1(2+w12+w22)w2(2+w12+w22)1+3w12+3w22+w14+w24+2w12w22

The Left Cauchy-Green tensor is a second order tensor and symmetric, then it has three principal strain invariants I1 ,I2 , and I3 given as

I1 = trace B,   I2= 12[trB2- trB2]= trace B-I ,       I3= det B

I1=3+w12+w22          I2=3+w12+w22=I1 , I3=1                    (4)

The third strain invariant shows that the material is incompressible,  since I3=1

Using the Neo-Hookean strain energy function material which is given by

W=μ2(I1 -3)                                                                                            (5)

where μ is the shear modulus

W1=∂W∂I1 =  μ2           W2=∂W∂I2=0                                                                (6)

- Stress tensor τ: The stress tensor for incompressible material is given by

τ=-ρI+2W1B-2W2B-1                                                                      (7)

where  is the unit tensor and ρ is the hydrostatic pressure

τ=-ρ000-ρ000-ρ+μ0μw10μμw2μw1μw2μ+μw12+μw22

Simplifying further, we have

τ=-ρ+μ0μw10-ρ+μμw2μw1μw2-ρ+μ+μw12+μw22                                        (8)

Representing stress tensor in Cartesian co-ordinate, form we have

τ=τxxτxyτxzτyxτyyτyzτzxτzyτzz                                                                                      (9)

Comparing (8) and (9) we obtain

τxx=-ρ+μ                                                                                                (10a)

τxy=τyx=0                                                                                                (10b)

τxz=τzx= μw1                                                                                          (10c)

τyy=-ρ+μ                                                                                                (10d)

τyz=τzy= μw2                                                                                          (10e)

τzz=-ρ+μ+μw12+μw22                                                                      (10f)

Here we need to transform the field equations to cylindrical polar coordinates (r,θ, z) since the problem is better handled in cylindrical polar co-ordinates.

The Cartesian co-ordinate is related to cylindrical polar co-ordinate by the relation given below

X1=rcosθ,  X2=rsinθ  ,  X3=Z

r=(X12+X22)12           θ=tan-1X2X1

That is from w(X1, X2) to w=w(r, θ)

From total differentiation

∂w(r, θ)=∂w(r, θ)∂r∂r+∂w(r, θ)∂θ∂θ

since r and θ are functions of  X1 and X2 then we have

∂w(r, θ)∂X1=∂w(r, θ)∂r∂r∂X1+∂w(r, θ)∂θ∂θ∂X1                                                                     (11)

∂r∂X1=cosθ and   ∂θ∂X1=-sin θr

Then (11) becomes

w1=  wrcosθ-wθsin θr                                                                                (12)

Squaring both sides of (12)

w12=   cos2θ(wr)2-2rcosθsinθwrwθ+1r2 sin2θ(wθ)2

∂w(r, θ)∂X2=∂w(r, θ)∂r∂r∂X2+∂w(r, θ)∂θ∂θ∂X2                                                                     (13)

∂r∂X2=sinθ      and   ∂θ∂X2=cosθr

w2=wrsinθ+wθcosθr                                                                                    (14)

Squaring both sides of (14)

w22=   sin2θ(wr)2+2rcosθsinθwrwθ+1r2 cos2θ(wθ)2

From (10f) that is  τzz=-ρ+μ+μw12+μw22

-ρ+μ1+w12+w22=-ρ+μ(1+cos2θ(wr)2-2rcosθsinθwrwθ+1r2 sin2θ(wθ)2+ sin2θ(wr)2+2rcosθsinθwrwθ+1r2cos2θ(wθ)2)

-ρ+μ1+w12+w22=-ρ+μ(1+(wr)2+1r2(w,θ)2)

using the value of w1,  w2 , w12 and w22 in equation (10) we obtain

τxx=τyy=μ-ρ

τxy =τyx ,=0

τxz =τzx=μ(cosθwr-sin θrwθ)

τyz=τyz=μ(sinθwr+cosθrwθ)

τzz=μ(1+(wr)2+1r2(wθ)2)-ρ

The coordinate transformation between cylindrical polar coordinate and Cartesian coordinate is given by

τ*=QτQT                                                                                                                         (15)

where Q is an orthogonal transformation tensor and QT is the transpose. Then (15) becomes

τ*=cosθsinθ0-sinθcosθ0001τxxτxyτxzτyxτyyτyzτzxτzyτzzcosθ-sinθ0sinθcosθ0001

τ*=cosθτxx+sinθτyxcosθτxy+sinθτyycosθτxz+sinθτyx-sinθτxx+cosθτyx-sinθτxy+cosθτyy-sinθτxz+cosθτyzτzxτzyτzzcosθ-sinθ0sinθcosθ0001                        (16)

using (10b) in equation (16), we obtain

τ*=cos2θτxx+sin2θτyy-cosθsinθτxy+cosθsinθτyycosθτxz+sinθτyz-cosθsinθτxx+cosθsinθτyysin2θτxx+cos2θτyy-sinθτxz+cosθτyxcosθτzx+sinθτzy-sinθτzx+cosθτzyτzz     (17)

In components form of the polar cylindrical material

τ*=τrrτrθτrzτθrτθθτθzτzrτzθτzz                                                                                                     (18)

Comparing (17) and (18), we have

τrr=cos2θτxx+sin2θτyy                                                                                             (19a)

τrθ=τθr=-cosθsinθτxy+cosθsinθτyy                                                                      (19b)

τθθ=sin2θτxx+cos2θτyy                                                                                            (19c)

τrz=τzr= cosθτxz+sinθτyz                                                                                      (19d)

τzθ=-sinθτzx+cosθτzy                                                                                              (19e)

τzz=τzz                                                                                                                            (19f)

Using equation (10) in (19), we have

τrr=μ-ρ                                                                                                                       (20a)

τθθ=μ-ρ                                                                                                                                 (20b)

τrθ=τθr=o                                                                                                                   (20c)

τrz=τzr=μwr                                                                                                               (20d)

τθz=τzθ=μrwθ                                                                                                              (20e)

τzz=μ(wr)2+1r2(wθ)2-ρ                                                                                       (20f)

- Equation of Equilibrium

The equilibrium equation is given by

Divτ*=0                                                                                                                                (21)

In component form, we have

∂τrr∂r+1r∂τrθ∂θ+∂τrz∂z+1rτrr-τθθ+Br=0                                                                             (22a)

∂τθr∂r+1r∂τθθ∂θ+∂τθz∂z+2rτrθ+Bθ=0                                                                                    (22b)

∂τzr∂r+1r∂τzθ∂θ+∂τzz∂z+1rτzr+Bz=0                                                                                            (22c)

where Br, Bθ and Bz are components of the body forces  in r, θ and z vectors respectively. The non-zero component of equilibrium equation is the axial component given by

∂τzr∂r+1r∂τzθ∂θ+∂τzz∂z+1rτzr+Bz=0                                                                                        23

In the absence of body force (23) reduces to

∂τzr∂r+1r∂τzθ∂θ+∂τzz∂z+1rτzr=0

where ∂τzr∂r=μwrr,    ∂τzθ∂θ=μrwθθ,  ∂τzz∂z=0

we obtain the equilibrium equation as

μwrr+μrwr+μr2wθθ=0

μ(wrr+1rwr+1r2wθθ)=0                                                                                                 (24)

where μ≠0

then equation (14) becomes

wrr+1rwr+1r2wθθ=0

r2wrr+rwr+wθθ=0

r2wrr+wθθ=-rwr                                                                                                          (25)

The standard monge’s equation is

R*r*+Ss+Tt*=V                                                                                                          (26)

Comparing (27) and (28) we have

R*=r2,  T=1,  S=0,  V=-rwr                                                                                   (27)

Any equation that satisfies (26) must satisfy

R*dpdt+Tdqdr-Vdrdt=0                                                                                                     (28a)

R*(dt)2-Sdrdt+Tdt2=0                                                                                          (28b)

Then (28a) and (28b) becomes

r2dpdθ+dqdR+rwrdrdθ=0                                                                                      (29a)

r2dθ2+dr2=0                                                                                                          (29b)

From (29b) , we have

dr=±-r2dθ2 =±irdθ                                                                                                (30)

Substituting (30) in (29a), we obtain

r2dpdθ+ irdqdθ+ rwrdrdθ=0                                                                                       (31a)

r2dpdθ- irdqdθ+ rwrdrdθ=0                                                                                    (31b)

Adding (31a) and (31b)  gives

2r2dpdθ+2 rwrdrdθ=0

Factoring dθ,  where dθ≠0 then we have

r2dp+ rwrdr=0

Let p=wr  where  w=w(r, θ)

r2dp=-rpdr

dpp=-drr

Inp=-Inr+InHθ

where Hθ>0 and θ≥0

p=Hθr

wr=Hθr

∂w∂r=Hθr

w= HθInr+Fθ                                                                                                           (32)

where r>0

- Use of boundary conditions

wa,θ=0,   wb, θ=kbsinθ                                                                             (33)

where k is a constant which is the magnitude of shear

Using equation (33) in (32) gives (34) and (35)

0= HθIna+F(θ)                                                                                              (34)

kbsinθ= HθInb+F(θ)                                                                                     (35)

Solving for F(θ) and Hθ, we obtain

Hθ=kbsin θ   In(α)    and    Fθ=-kbsinθInαIna                                                            (36)

where α=ba and α>0 since b>a

then (32) gives

wr,θ=λsinθInα Inra                                                                                                  (37)

Equation (37) satisfies the above boundary conditions given in equation (33).

wr=λsinθrInα

wΘ=λcosθInα Inra

where kb=λ

The stress components become

τrr=τθθ=μ-ρ

τrθ=τθr=o

τrz=τzr=μλsinθInα1r

τθz=τzθ=μrλcosθInα Inra

τzz=μ(λsinθInα1r)2+1r2(λcosθInα Inra)2-ρ

- Conclusion

This present work establish an exact analytical solution for the angular displacement and stresses for the anti-plane shear deformation of an incompressible Hollow cylinder made of Neo-Hookean material. Equation (37) gives the displacement. Finally, we obtain the components of the stress at any cross section of the cylinder made of Neo-Hookean material.

References

- Erumaka E. N (2003): Finite anti plane shear deformation of a class of Ogden containing a line crack. Journal of Mathematical Sciences 14 (2) 65-72.

- Erumaka E. N and Onugha  E. E (2013): Stresses and deformation in a Neo-Hookean Half space deforming under Anti plane shear loading. Journal of the Nigeria Association of Mathematics Physics. 25, 77-80.

- Polygnone D. A and Horgan C. O (1992): Axisymmetric finite Anti plane shear of compressible Non linearly elastic circular Tubes. Quart. Appl. Math. 50, 323-341.

- Ejike U. B. C. O and Erumaka U. N (2006): Finite deformation of a rotating circular cylinder of Blatz-ko material. Journal of Mathematical Science 17 (1), 63-71.

- Simmand J. C and Warne P (1992): Azimuthal shear of compressible non-linaerly Elastic orthotropic Tubes of finite Extent. Int. J. Non Linear Mech. 27. 447-464.

- Horgan C. O and Giuseppe Saccomandi (2001): Anti-plane shear deformations for non-Gaussian isotropic, incompressible hyperelastic materials. Proceeding of the Royal Society of London. 457 1999-2017.

- Horgan C. O and Giuseppe Saccomandi (2003): Superposition of Generalized plane strain on anti-plane shear deformations in isotropic incompressible Hyperelastic. Journal of Elastics 73, 221-235

- Lejla Sultanova (2018): Two parametric analysis of Anti-plane shear deformation of a coated Elastic Half-Space. Acta Mechanic et automatic 12. 270-275.

- Horgan C. O (1995): Anti-plane shear deformations in linear and non linear solid mechanics. SIAM review 37 (1) 53-81.

- Yang F. (2006): Asymptotic solution to Axisymmetric indention of a compressible elastic thin film. Thin solid films, 515 (4), 2274-2283.

- Darijani H and Bahremen M  (2015);  New polynomial strain energy function; application to rubbery circular cylinders under finite extension and torsion, Journal of applied polymer science, Vol 132.

- Robert M., Destrade M. and Saccomandi G.(2016); Strain energy function for isotropic non-linear elastic incompressible solids with linear finite strain response in shear and torsion, Extreme Mechanics Letters 9 , 204–206

- Merodio J. and  Ogden R.W. (2015) ; Extension , inflation and torsion of a residually stressed circular  cylindrical tube , continnum mechanics thermodynamics  28: 157 -174.

- Anani  Y. and Gholamhosein R. (2015); Stress analysis of thick pressure vessel composed of incompressible hyperelastic materials;  International Journal of Recent advances in Mechanical Engineering (IJMECH),  Vol.4, No.3

- Shearer T., Abrahams I. D. and Parnell W.J. 9 (2013); Torsional wave propagation in a pre-stressed hyperelastic annular circular cylinder, Quaterly journal of mechanics and applied Mathematics 66, 4, 465-487.

- Grine F, Saccomandi G, Arfaoui M, (2020): Elastic machines: a non standard use of the axial shear of linear transversely isotropic elastic cylinders. Int. Journal of solids structure 185, 57-64.

- Marco coco and Giuseppe saccomandi (2022): Superposing plane strain on anti-plane shear deformations in a special class of fibre-reforced incompressible hyperelastic materials. International Journal of Solids and structures, 256.