Resource Valorization of Sericulture Materials for Value-Added Products

Abstract

Sericulture produces a considerable amount of waste during various stages of silk production, including mulberry residues, silkworm litter, pupae, and reeling by-products, which may lead to environmental problems if not properly managed. However, most existing research has primarily focused on silk fiber production, while the potential of sericulture waste as a source of bioactive compounds, biomaterials, and industrial raw materials remains insufficiently explored. This review aims to examine the potential of these by-products for resource valorization and the development of value-added products. The analysis shows that these materials are rich in nutrients and bioactive compounds and can be utilized in applications such as animal feed, organic fertilizers, vermicompost, biogas production, and recycled silk products. Overall, the study emphasizes the scientific and practical significance of converting sericulture waste into sustainable products to reduce environmental pollution, improve resource efficiency, and enhance economic returns in the sericulture sector.

Keywords: Sericulture, Silkworm, Waste, Bombyx mori

- Introduction

Silkworms, Bombyx mori, belong to the order Lepidoptera and the family Bombycidae. This insect is considered economically important because it produces silk by spinning a protective cocoon around its body (Hussain et al., 2011). In India, the major producers of mulberry silk include Karnataka, Andhra Pradesh, Tamil Nadu, Jammu & Kashmir, and West Bengal, while non-mulberry silk is mainly produced in states such as Jharkhand, Chhattisgarh, Odisha, and the northeastern region, particularly Assam (Mariyappanavar, 2024).

Sericulture is an agro-based and labor-intensive activity that provides employment opportunities for many people in both rural and urban areas. The cultivation of mulberry and non-mulberry host plants is essential for silk production. Among these, mulberry is the most widely used host plant for silkworm rearing across the world. Apart from its role in sericulture, mulberry is also valued for its medicinal properties, tasty fruits, use in landscaping, and as animal fodder (Singhal et al., 2005a, 2005b).

Sericulture and agriculture are closely connected, as the quality of silk largely depends on proper cultivation of host plants and efficient silkworm rearing practices. During different stages of silkworm rearing and silk processing, several types of waste are generated, including silkworm castings, damaged cocoons, pupal shells, and other by-products. However, these materials can be effectively utilized for various economic and value-added purposes (Mariyappanavar, 2024).

The sericulture industry generates three main categories of waste: surplus mulberry plant materials such as twigs, leaves, and fruits; waste produced during cocoon reeling, including silkworm pupae; and bed waste generated during the rearing of silkworms. A significant portion of this waste originates from mulberry cultivation. Studies have shown that nearly 45% of the mulberry leaves consumed by silkworms are later discharged as waste. On average, marginal farmers produce around 250–300 kg of such waste, which has the potential to be converted into approximately 2500 kg of organic manure, sufficient to fertilize about 0.067 hectares of agricultural land (Wenhua, 2001).

This review examines the diverse utilisation pathways of sericulture-derived waste, highlighting their potential to generate both economic value and social benefits. Materials conventionally classified as waste within sericultural systems may, in fact, be repurposed into valuable resources. Silk, produced by silkworms, consists predominantly of natural proteins, chiefly fibroin and sericin. Fibroin serves as the principal structural protein, conferring strength, stiffness, and overall mechanical integrity to the silk fibre (Cui et al., 2018; Koh et al., 2015). In contrast, sericin, which accounts for approximately 25–30% of the total silk protein, envelops the fibroin filaments in a cohesive matrix, facilitating their adhesion and contributing to the formation of the cocoon structure (Cao & Zhang, 2016).

During silk processing, especially in the reeling stage, most of the sericin is removed to produce raw silk. As a result, large quantities of sericin are released into wastewater from silk processing units, which may contribute to environmental pollution. However, sericin and other silk-related wastes have the potential to replace or complement synthetic materials in several applications. One promising approach is the conversion of silk waste into bio-composite materials. Proper waste management and innovative utilization strategies can therefore transform sericulture waste into valuable products with significant practical applications (Qayoom & Manzoor, 2024).

- Types of Sericulture waste and their composition:

  - Mulberry waste

Mulberry (Morus spp.) constitutes the primary host plant for the silkworm (Bombyx mori), and its cultivation generates a substantial volume of plant-derived residues through routine agronomic and harvesting practices. These by-products comprise pruned branches, stems, roots, bark, senescent or unsuitable leaves, leaf midribs, fallen foliage, as well as overripe or abscised fruits. Standard management operations, including seasonal pruning for canopy regulation, the removal of aged foliage, and general plantation upkeep, contribute significantly to the accumulation of lignocellulosic biomass. Furthermore, discarded fruits and damaged leaves exacerbate the overall volume of plant waste produced (Kandari et al., 2019). Empirical evidence suggests that a single hectare of mulberry cultivation can yield approximately 4–5 tonnes of dry pruned biomass annually (Rathore et al., 2018).

  - Silkworm rearing waste

Silkworm faecal matter, commonly termed silkworm excreta or litter, represents a major by-product generated during the larval rearing phase. It predominantly comprises partially digested residues of mulberry leaves, fragments of plant cell walls, and associated microbial biomass. Intensive silkworm cultivation yields considerable quantities of this material, with estimates indicating the production of approximately 60–70 kg of faecal waste per 100 disease-free layings (DFLs) (Ahmed et al., 2020). Historically, such waste was largely discarded or utilised only minimally as manure. However, it is increasingly being recognised as a valuable resource with potential applications across agriculture, pharmaceuticals, and a range of bioresource-based industries.

  - Cocoon waste

Pierced or cut cocoons are predominantly generated during grainage operations, wherein silkworms are permitted to emerge as moths for breeding purposes. In contrast to intact cocoons utilised in silk reeling, these cocoons are compromised by the emergence aperture created by the moth, which disrupts the continuity of the silk filament. As a result, they are unsuitable for reeling into raw silk. Nevertheless, they remain a rich source of silk proteins, notably fibroin and sericin, thereby retaining potential for diverse value-added applications (Kumaresan & Sannappa, 2013). In India, several thousand kilograms of pierced cocoons are produced annually. Despite their considerable potential for higher-value utilisation, a substantial proportion of this material continues to be channelled into spun silk industries at comparatively low market prices (Rathore et al., 2018).

  - Reeling and processing waste

Silk reeling waste comprises fibrous residues produced during the unwinding of silk filaments from cocoons in reeling operations. This category of waste includes floss silk, pelade, pierced or cut cocoons, reeling noil, and short fibres that are incapable of forming continuous yarn. It further encompasses unreelable portions of cocoons, broken filaments generated during reeling, and residual sericin remaining after the degumming process (Kumaresan et al., 2019). In commercial silk production systems, such reeling waste may constitute approximately 25–30% of the total cocoon mass (Zhou et al., 2020).

Silkworm pupae represent another significant by-product of the silk industry, obtained following the extraction of silk from cocoons. During industrial reeling, cocoons are subjected to boiling to soften sericin, thereby facilitating the unwinding of continuous silk filaments. The residual pupae, typically accounting for 50–55% of the cocoon weight, are subsequently recovered as a secondary product (Kumar & Rajkhowa, 2019). Large-scale silk production generates substantial quantities of pupae; for example, India produces in excess of 25,000 tonnes annually, a considerable proportion of which remains underutilised despite its notable nutritional and industrial potential (Zhang et al., 2021).

  - Other waste products:

Chrysalis remnants (pupal exuviae) and adult moth bodies constitute ancillary by-products of sericulture, predominantly generated during seed production (grainage), wherein silkworms are permitted to complete their life cycle and emerge as adult moths. Following mating and oviposition, the spent moths are generally discarded, and in certain instances, the empty pupal casings are segregated from other grainage residues. Despite being frequently overlooked, these materials are enriched with biopolymers and bioactive compounds, thereby presenting considerable potential for utilisation across industrial, pharmaceutical, and agricultural sectors (Rajkhowa et al., 2013).

- Circular Economy Models in Sericulture:

The circular economy model promotes the reuse and recycling of all sericulture by-products within the production cycle, helping to reduce environmental pollution while increasing economic benefits (Qayoom & Manzoor, 2024). For instance, cocoon shells and sericin extracted during degumming can be utilized in biomedical, cosmetic, and industrial applications (Altman et al., 2022; Menaka et al., 2025). Mulberry leaf residues and silkworm rearing bed waste can be converted into compost to enrich soil for organic mulberry cultivation, whereas silkworm pupae can be used as livestock feed or for nutraceutical products. By reintegrating these waste streams into the production process, sericulture can operate as a closed-loop system, optimizing resource use and reducing environmental impact. This strategy also generates multiple income sources for farmers, supporting rural livelihoods and promoting sustainable practices.

- Utilization of sericulture waste

  - Waste as Vermicomposting:

Vermicomposting is the process of converting organic waste into nutrient-rich compost with the help of earthworms (Lim et al., 2016; Lee et al., 2018). By-products from silkworm rearing can serve as feedstock for vermicomposting, while silkworm litter alone can be used in pharmaceutical applications or for biogas production (Sharma et al., 2022a, 2022b). Annually, one hectare of mulberry cultivation generates approximately 15 tonnes of sericulture waste, containing around 280-300 kg of nitrogen, 90-100 kg of phosphorus, and 150-200 kg of potash from silkworm rearing and farm residues (Das et al., 1997).

According to Altman et al. (2022) and Menaka et al. (2025), one of the main applications of surplus mulberry leaves is composting. Through composting or vermicomposting, these leaves are converted into organic manure rich in nutrients, which enhances soil fertility, stimulates microbial activity, and promotes sustainable agricultural practices (Dhakal et al., 2024; Qayoom & Manzoor, 2024). In addition, mulberry leaves can be dried and processed into animal feed, offering a high-protein supplement for livestock, poultry, and goats, thereby improving growth and productivity (Hassan, 2025). Beyond agricultural uses, mulberry leaf extracts are employed in herbal supplements and functional foods due to their medicinal properties, including anti-diabetic, antioxidant, and immune-boosting effects, highlighting the significant value of this sericulture by-product (Menaka et al., 2025).

  - Waste in Value-addition

Sericulture waste, once seen primarily as an environmental issue, is now increasingly viewed as a valuable resource for producing high-value products. Transforming these by-products into economically and socially useful materials not only addresses waste disposal problems but also generates additional income for farmers and rural entrepreneurs. Studies by Singha et al. (2024), Kaushik et al. (2025), and Prajapati et al. (2022) indicate that effective use of sericulture residues-including mulberry leaves, silkworm pupae, defective cocoons, and silk sericin can promote sustainable rural livelihoods while supporting circular bioeconomy initiatives. Such advancements in waste management are helping to make the sericulture industry more resource-efficient and environmentally sustainable.

  - Silkworm pupae

Defatted silkworm pupae meal represents a highly nutritious protein source suitable for livestock, poultry, and aquaculture species. Studies have demonstrated that replacing up to 40% of soybean meal with pupae meal in poultry diets supports comparable growth performance while improving feed conversion efficiency (Dong et al., 2018). In aquaculture, inclusion of pupae meal has been shown to enhance fish growth, increase feed digestibility, and improve flesh quality (Wang et al., 2020). Its elevated lipid content also supplies essential fatty acids that are beneficial for aquatic organisms. In several Asian countries, including China, Korea, Thailand, and Vietnam, silkworm pupae are traditionally consumed in various forms, such as boiled, fried, roasted, or incorporated into snacks, soups, and sauces (Rumpold & Schlüter, 2013). Furthermore, pupal oil extracted via mechanical or solvent-based methods is rich in polyunsaturated fatty acids and finds applications in cosmetics, skincare formulations, and soap production (Kumar & Rajkhowa, 2019).

  - Defective cocoons

Around 5–10% of the total cocoon production during silk reeling is classified as defective or leftover (Menaka et al., 2025; Dhakal et al., 2024). These cocoons can be processed into silk waste and spun into yarn, which is then used to produce lower-grade silk textiles and handicrafts. Although spun silk is of lower quality compared to filament silk, it finds wide application in textiles, embroidery, and home furnishings. Additionally, damaged cocoons can be creatively repurposed into decorative items such as garlands, ornamental vases, and other craft products, providing opportunities for rural entrepreneurship and cottage industries (Qayoom & Manzoor, 2024; Hassan, 2025). Such initiatives not only generate economic value but also help preserve traditional craft skills.

  - Silk Sericin

Sericin is widely utilized in cosmetic products such as skin moisturizers, anti-aging creams, and hair conditioners because of its ability to form a protective film, retain moisture, and provide antioxidant benefits. It enhances skin elasticity and smoothness while adding shine and manageability to hair by maintaining hydration (Kato et al., 1998). Additionally, sericin recovered from degumming wastewater can be reapplied to fabrics, where it imparts antimicrobial activity, UV protection, and moisturizing properties, thereby increasing the functional value of silk and silk-blend textiles (Yuan et al., 2020).

- Benefits of waste utilization in sericulture:

Effective use of sericulture waste provides important economic and environmental benefits, contributing to the sustainability of the industry and supporting rural livelihoods. Proper handling and value addition of by-products such as mulberry leaves, silkworm pupae, defective cocoons, and silk sericin can create additional income opportunities while also encouraging environmentally responsible practices (Menaka et al., 2025).

  - Economic benefits

According to Altman et al. (2022), farmers can increase their income beyond traditional silk production by converting sericulture waste into value-added products such as organic manure, animal feed, pupae oil, spun silk, and sericin-based cosmetic products. This diversification helps improve financial security by reducing dependence solely on cocoon sales, especially during periods when silk market prices fluctuate (Menaka et al., 2025). In addition, the utilization of sericulture waste contributes to employment generation. Activities such as composting, vermicomposting, extraction of pupae oil, production of sericin, and development of handicrafts create job opportunities in rural areas, particularly benefiting women and small-scale farmers (Hassan, 2025; Qayoom & Manzoor, 2024).

  - Environmental Benefits:

The effective use of sericulture waste plays an important role in reducing environmental problems associated with traditional disposal practices. Conventional methods, such as dumping mulberry leaves, pupae, and cocoon residues in landfills or water bodies, can cause soil and water pollution. Converting these by-products into useful materials such as compost, animal feed, or other industrial products helps decrease waste accumulation and reduces environmental contamination (Dhakal et al., 2024; Qayoom & Manzoor, 2024).

Composting and vermicomposting of mulberry leaves and silkworm rearing bed waste also contribute to improved soil quality by increasing nutrient availability, microbial activity, and organic matter content. Studies have shown that soils treated with organic compost derived from sericulture residues exhibit greater fertility and improved structure compared to soils that rely solely on chemical fertilizers (Menaka et al., 2025). Moreover, this practice decreases dependence on synthetic fertilizers, thereby reducing environmental risks such as groundwater pollution and soil degradation (Hassan, 2025).

Table 1. Major sericulture by-products and their applications

| By- products | Utilization | References |
| --- | --- | --- |
| Silkworm pupae | Animal feed, fish feed, oil extraction, protein powder | Rahman et al., 2013 |
| Mulberry leaves waste | Organic manure, compost, biogas production | Ullal & Narasimhanna 1978 |
| Silkworm litter | Biofertilizer and vermicompost | Jolly, 1987 |
| Reeling waste / silk waste | Production of spun silk, handicrafts, textiles | Gulrajani, 1992 |
| Sericin (silk protein) | Cosmetics, pharmaceuticals, biomedical materials | Aramwit et al., 2010 |

- Conclusion

Sericulture waste, including mulberry residues, silkworm litter, pupae, and defective cocoons, is a valuable resource rather than just an environmental burden. Proper utilization through composting, animal feed, spun silk production, and extraction of sericin and fibroin reduces pollution, adds economic value, and supports rural livelihoods. Integrating these practices into sericulture promotes a circular economy, sustainable agriculture, and resource-efficient silk production, making the industry environmentally and economically resilient.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

Ahmed, S., Hossain, M. and Rahman, M. 2020. Utilization of sericulture waste for sustainable agriculture: Nutrient potential and composting techniques. Journal of Environment Management, 270:110892.

Altman, G. H., & Farrell, B. D. 2022. Sericulture as a sustainable agroindustry. Cleaner and Circular Bioeconomy, 2: 100011.

Aramwit, P., Kanokpanont, S., Nakpheng, T. and Srichana, T. 2010. The effect of sericin on wound healing. Journal of Biomaterials Science, 21(15): 217–225.

Cao, T. and Zhang, Y. 2016. Processing and characterization of silk sericin from Bombyx mori and its application in biomaterials and biomedicines. Materials Science & Engineering: C, 61:940-952.

Cui, Y., Zhu, Y., Lin, Y., Chen, L., Feng, Q., Wang, W. and Xiang, H., 2018. New insight into the mechanism underlying the silk gland biological process by knocking out fibroin heavy chain in the silkworm. BMC genomics, 19(1):215.

Das, P. K., Bhogesha, K., Sundareswaran, P., Madhana Rao, Y. R. and Sharma, D. D. 1997. Vermiculture: scope and potentiality in sericulture. Indian Silk, 36(2): 23-26.

Dhakal, R., Sahu, R. K., Das Gupta, D., Saikia, M., Borthakur, S., Majumder, M., Mech, S., Thapa, M., Dutta, P., and Kalita, J. 2024. Recycling of protein rich silk industry waste for potential food and therapeutic application. Food Bioscience, 60:104461.

Dong, X.F., Gao, W.W., Tong, J.M. and Xie, M. 2018. Effect of replacing soybean meal with silkworm pupae meal on growth performance, serum biochemical indices and immunity of broilers. Animal Feed Science & Technology, 240:34-41.

Gulrajani, M. L. 1992. Degumming of silkmoth cocoons. Review Programme in Colouration, 22: 79-89.

Hassan, R. 2025. Transitioning the silk industry towards circularity. Journal of Textile Sustainability, 8(1), 22 30.

Hussain, M., Khan, S. A., Naeem, M., and Nasir, M. F. 2011.  Effect of Rearing Temperature and Humidity on Fecundity and Fertility of Silkworm, Bombyx mori L. (Lepidoptera: Bombycidae). Pakistan Journal of Zoology. 43(5): 979-985.

Jolly, M.S. 1987. Appropriate Sericulture Techniques. Gitanjali printers, Bangalore, 63-106.

Kandari, L.S., Thakur, A.K. and Negi, G.C.S. 2019. Utilization of mulberry plant parts: A review on its traditional and industrial applications. Indian Journal of Sericulture, 58(2): 101-109.

Kato, N., Sato, S., Yamanaka, A., Yamada, H. and Fuwa, N. 1998. Silk protein, sericin, inhibits lipid peroxidation and tyrosinase activity. Bioscience, Biotechnology & Biochemistry, 62(1): 145-147.

Kaushik, A., Saikia, S., Gogoi, C., & Kaushiki, B. 2025. Circular Strategies and Eco Innovations for Sustainable Sericulture Waste Management. Journal of Experimental Agriculture International, 47(8): 569–580.

Koh, L.D., Cheng, Y., Teng, C.P., Khin, Y.W., Loh, X.J., Tee, S.Y., Low, M., Ye, E., Yu, H.D., Zhang, Y.W. and Han, M.Y. 2015. Structures, mechanical properties and applications of silk fibroin materials. Progress in Polymer Science, 46: 86-110.

Kumar, D. and Rajkhowa, R. 2019. Utilization of silk industry byproducts: A review. Journal of Textile Science & Engineering, 9(4): 1-8.

Kumaresan, P. and Sannappa, B. 2013. Utilization of pierced cocoons for value-added products in sericulture. Indian Journal of Sericulture, 52(1): 34-39.

Kumaresan, P., Sannappa, B. and Kumar, D. 2019. Recycling of silk reeling waste into spun silk yarn: A sustainable approach. Textile Trends, 62(7): 45-49.

Lee, L.H., Wu, T.Y., Shak, K.P.Y., Lim, S.L., Ng, K.Y., Nguyen, M.N. and Teoh, W.H. 2018. Sustainable approach to biotransform industrial sludge into organic fertilizer via vermicomposting: A mini‐review. Journal of Chemical Technology & Biotechnology, 93(4): 925-935.

Lim, S. L., Lee, L. H., & Wu, T. Y. 2016. Sustainability of using composting and vermicomposting technologies for organic solid waste biotransformation: recent overview, greenhouse gases emissions and economic analysis. Journal of cleaner production, 111: 262-278.

Mariyappanavar, S. 2024. Impact of sericultural practices on waste generation and its management in India. JETIR, 11:732.

Menaka, S., Sabarish, M., Durgadevi, R., Kalpana, R., Anusuya, G., Bhuvaneshwari, T. and Brindhabharathi, S. A. 2025. Sericulture by-products: A comprehensive review on utilization and sustainable development. International Journal of Research in Agronomy, 8(8S): 447- 454.

Prajapati, D., Zhou, F., Dwivedi, A., Singh, T., Lakshay, L. and Pratap, S. 2022. Sustainable Agro-Food Supply Chain in E Commerce: Towards the Circular Economy. Sustainability, 14(14): 8698.

Qayoom, K. and Manzoor, S. 2024. Recent Sustainable Practices for Eco-Friendly Utilisation of Sericultural Waste Products. Journal of Environmental Agriculture and Innovation, 12(3): 45-58.

Rahman, M., Khan, M. and Islam, M. 2013. Utilization of silkworm pupae as a source of nutrients. Journal of Insect Science, 13(1): 1–8.

Rajkhowa, R., Kafi, A.A. and Wang, X. 2013. Silk fibroin/polymer composites for advanced applications. Materials Research Bulletin, 48(9): 3523-3533.

Rathore, S., Sharma, V. and Kumar, S. 2018. Value addition of sericulture by-products for sustainable livelihood. Indian Journal of Sericulture, 57(1): 45-51.

Rumpold, B.A. and Schlüter, O.K. 2013. Nutritional composition and safety aspects of edible insects. Molecular Nutrition & Food Research, 57(5): 802-823.

Sharma, A., Gupta, R.K., Sharma, P., Qadir, J., Bandral, R.S. and Bali, K. 2022. Technological innovations in sericulture. International Journal of Entomology Research, 7(1): 7-15.

Sharma, V., Rattan, M. and Chauhan, S.K. 2022. Potential use of sericultural by-products: a review. Pharma Innovation Journal, 11(8): 1154-1158.

Singha, T. A., Bhattacharyya, B., Gogoi, D., Bora, N., Marak, M., & Borgohain, A. 2024. Innovative Utilisation of Sericulture Resources to Value Added Products – A Review. Journal of Advances in Biology & Biotechnology, 27(9): 1302 1309.

Singhal, B.K., Dhar, A., Khan, M.A. and Bindroo, B.B. 2005a. Utilization of sericultural by-products as urgent need for sustainable sericulture. In: Progress of Research in Organic Sericulture and Seri Byproduct Utilization. Seri Scientific Publishers.

Singhal, B.K., Dhar, A., Khan, M.A., Sengupta, D. and Dhar, S.L. 2005b. Mulberry by-products utilization for sustenance of sericulture industry of Jammu and Kashmir. Proceedings of the 20th Congress of the International Sericultural Commission, 152-156.

Ullal, S. R. and Narasimhanna, M. N. 1978. Handbook of practical sericulture. Central Silk Board.

Wang, X., Li, D., Xu, Y. and Zhang, J. 2020. Effects of silkworm pupae meal on growth performance, body composition and digestive enzyme activity of juvenile grass carp (Ctenopharyngodon idella). Aquaculture Research, 51(5):2073- 2083.

Wenhua, L.  2001.  Agro-ecological Farming Systems in China (Man and the Biosphere Series). Chinese Academy of Sciences. Beijing, China: UNESCO.

Yuan, J., Zhang, L. and Dong, C. 2020. Functionalization of textiles with sericin recovered from silk degumming wastewater. Journal of Cleaner Production, 255:120215.

Zhang, Y., Sun, T. and Zhao, L. 2021. Nutritional evaluation of silkworm pupae and their potential applications. Food Reviews International, 37(3):273-289.

Zhou, X., Li, Y., Chen, H. and Wu, W. 2020. Biodiesel production from silkworm pupae oil: Process optimization and fuel properties. Renew Energy, 146:2085-2093.