Original Research Article

Data-Driven Statistical Assessment of Ambient Air Quality Parameters in Agartala Smart City vis-à-vis Guwahati and Delhi

Aim: To conduct a comprehensive data-driven statistical assessment of ambient air quality parameters in Agartala Smart City, with comparative analysis of Guwahati and Delhi, representing different levels of urbanization, emission intensity, and meteorological variability.

Study Design: The study adopts a comparative and analytical research design integrating environmental chemistry with statistical methods to examine spatial and temporal variations in air pollution.

Place and Duration of Study: The study focuses on three Indian cities—Agartala, Guwahati, and Delhi. Secondary data were collected for a defined study period from official monitoring agencies.

Methodology: The research considers major air quality indicators such as PM₂.₅, PM₁₀, NO₂, and SO₂, which are key determinants of the Air Quality Index (AQI) and public health risk. Data were obtained from the Central Pollution Control Board and respective State Pollution Control Boards. A structured statistical framework was employed, including descriptive statistics, coefficient of variation, correlation analysis, and time-series-based linear trend modeling. These techniques were used to evaluate pollutant concentration patterns, variability, interrelationships, and trends across the selected cities. Comparative statistical diagnostics were applied to identify regional disparities in atmospheric pollution characteristics.

Results: The findings reveal significant inter-city variation in air quality. Delhi exhibits consistently higher pollutant concentrations due to intense vehicular traffic, industrial emissions, and construction activities. Guwahati shows moderate pollution levels influenced by urban expansion and meteorological conditions. Agartala, although relatively less polluted, demonstrates increasing variability in particulate matter levels due to transportation, roadside dust, biomass burning, and seasonal effects. The coefficient of variation and correlation analysis indicate strong interdependence among pollutants, suggesting common anthropogenic sources and atmospheric transformation processes.

Conclusion: The study demonstrates that statistical analysis provides valuable insights into urban air pollution dynamics and regional variability. While Agartala currently maintains relatively better air quality, the observed increasing variability in pollutant levels indicates emerging environmental concerns. The findings highlight the need for proactive, region-specific air quality management strategies and reinforce the importance of data-driven approaches for sustainable urban environmental planning and policy formulation.

ABSTRACT

Keywords: Air Quality Index, Atmospheric Pollutants, Coefficient of Variation, Linear Trend Analysis, Sustainable Environmental Management.

- Introduction

Air pollution has become a major environmental and public health issue in India due to rapid urbanization, industrialization, increasing vehicular traffic, and infrastructure development. Urban air pollution is mainly characterized by high levels of particulate matter (PM₂.₅ and PM₁₀) along with gaseous pollutants such as nitrogen dioxide (NO₂) and sulphur dioxide (SO₂). Among these, particulate matter is the primary contributor to the Air Quality Index (AQI) and is associated with serious health risks including respiratory and cardiovascular diseases.

The Air Quality Index (AQI), developed by the Central Pollution Control Board (CPCB), is used to communicate overall air quality by combining multiple pollutants into a single value with defined health categories. In most Indian cities, PM₂.₅ and PM₁₀ significantly influence AQI levels, while SO₂ and NO₂ generally remain within permissible limits.

Delhi, the national capital, is one of the most polluted megacities in the world due to high vehicular density, industrial emissions, construction activities, road dust, and crop residue burning in neighbouring states. These factors frequently lead to “Very Poor” to “Severe” AQI levels.

Guwahati, the largest city in Northeast India, has experienced rapid urban growth, resulting in increasing pollution from vehicular emissions, road dust, and construction activities. Although its pollution levels are lower than Delhi, rising particulate concentrations have been observed.

Agartala, a Smart City under the Government of India’s Smart Cities Mission, has traditionally maintained relatively better air quality due to limited industrial activity. However, increasing vehicles and urban development are gradually contributing to higher pollution levels.

Meteorological factors such as rainfall, wind patterns, and temperature influence pollutant dispersion. While higher rainfall in Northeast India helps reduce pollution, increasing particulate levels in Guwahati and Agartala indicate growing anthropogenic impacts. Comparative studies between northeastern cities and megacities like Delhi remain limited.

- METHODOLOGY

  - Study Area

The present study focuses on three Indian cities—Delhi, Guwahati, and Agartala—which represent different levels of urbanisation and air pollution. These cities differ in geographical location, climate, population density, and economic activities, all of which influence their air quality conditions

    - Delhi 
Delhi is located in northern India between latitudes 28°24′17″ N to 28°53′00″ N and longitudes 76°50′24″ E to 77°20′37″ E, covering an area of about 1,483 km². Situated in the Indo-Gangetic Plain, the city experiences an extreme continental climate with very hot summers (up to 45–47°C) and cold winters (around 5–7°C). As one of the most densely populated megacities in the world, Delhi faces severe air pollution due to heavy vehicular traffic, industrial activities, construction work, road dust, and biomass burning. These factors contribute to very high levels of PM₂.₅ and PM₁₀, making Delhi a benchmark city for severe air pollution in India.

    - Guwahati 
Guwahati, the largest metropolitan city in Northeast India, is located at 26°10′ N latitude and 91°45′ E longitude and covers about 216 km². The city lies on the southern bank of the Brahmaputra River and is surrounded by hills, which influence local air circulation. It has a humid subtropical climate with moderate summers (30–35°C) and mild winters (10–15°C). Although rainfall helps reduce pollution, rapid urbanisation has increased emissions from vehicles, road dust, construction activities, brick kilns, and small industries. The surrounding hills sometimes restrict air movement, causing pollutant accumulation, especially during winter.

    - Agartala 
Agartala, the capital of Tripura, is located at 23°50′ N latitude and 91°16′ E longitude near the India–Bangladesh border and covers an area of about 76 km². The city experiences a tropical monsoon climate with warm summers (30–34°C) and mild winters (10–12°C). Compared to major Indian cities, Agartala has traditionally maintained relatively better air quality due to limited industrial activity. However, recent urban development under the Smart Cities Mission has led to increasing pollution from rising vehicle numbers, construction work, diesel generators, and urban expansion. This makes Agartala an important case for studying changing air quality in a developing Smart City.

| City | Urban Type | Pollution Level | Role in Study |
| --- | --- | --- | --- |
| Delhi | Megacity | Very High | National benchmark |
| Guwahati | Regional metro | Moderate and rising | Northeast urban hub |
| Agartala | Smart City | Low–moderate | Emerging urban system |

Table1. Compare in three City

  - Data Collection

We are collected the secondary data

    - Central Pollution Control Board (CPCB)

The data used in the present study were obtained from the official database of the Central Pollution Control Board (CPCB), Government of India. CPCB operates and supervises the National Air Quality Monitoring Programme (NAMP) and Continuous Ambient Air Quality Monitoring Stations (CAAQMS) across various cities, ensuring standardized and quality-controlled air pollution measurements. Secondary data on major atmospheric pollutants, including particulate matter (PM₂.₅ and PM₁₀) and gaseous pollutants such as nitrogen dioxide (NO₂) and sulfur dioxide (SO₂), were collected for Agartala, Guwahati, and Delhi from the CPCB online portal. The dataset consists of routinely monitored pollutant concentrations recorded at regular intervals and publicly made available for research and policy analysis.

The CPCB monitoring framework follows uniform sampling, calibration, and validation procedures, which enhance the reliability and comparability of pollutant measurements across different cities. The collected data were systematically organized, screened for completeness, and processed for statistical analysis in order to ensure consistency and accuracy in inter-city comparison. These officially reported datasets form the empirical basis of the present comparative assessment of atmospheric pollutants.

    - Tripura State Pollution Control Board (TSPCB)

For Agartala Smart City, atmospheric pollutant data were obtained from the Tripura State Pollution Control Board (TSPCB), which functions under the provisions of the Air (Prevention and Control of Pollution) Act, 1981 and the Environment (Protection) Act, 1986. TSPCB is responsible for monitoring ambient air quality within the state and operates air monitoring stations in coordination with national regulatory frameworks. The board regularly measures key atmospheric pollutants, including particulate matter (PM₂.₅ and PM₁₀) and gaseous pollutants such as nitrogen dioxide (NO₂) and sulfur dioxide (SO₂), following standardized monitoring and quality assurance protocols.

The data collected from TSPCB are generated through approved sampling methods and validated monitoring instruments, ensuring reliability and consistency. These officially reported datasets were used as secondary data for statistical analysis and comparative assessment in the present study.

    - Assam Pollution Control Board (APCB)

For Guwahati city, atmospheric pollutant data were collected from the Assam Pollution Control Board (APCB), the statutory authority constituted under the Water (Prevention and Control of Pollution) Act, 1974 and the Air (Prevention and Control of Pollution) Act, 1981. APCB is responsible for monitoring, controlling, and regulating environmental pollution within the state of Assam. The board operates ambient air quality monitoring stations under national guidelines and in coordination with the Central Pollution Control Board (CPCB).

The monitoring framework of APCB includes systematic measurement of major atmospheric pollutants such as particulate matter (PM₂.₅ and PM₁₀) and gaseous pollutants including nitrogen dioxide (NO₂) and sulfur dioxide (SO₂). The data are collected using standardized sampling techniques and calibrated instruments to ensure accuracy, consistency, and reliability. The officially reported secondary data from APCB were used in the present study for statistical analysis and inter-city comparative evaluation of pollutant concentration levels.

    - www.data.gov.in

    - aqi.in

    - IQAIR

  - METHOD

    - Data Collection

The air quality data used in this study were collected from the official database of the Central Pollution Control Board (CPCB) and Tripura State Pollution Control Broad and Assam Pollution Control Broad. The dataset includes daily average concentrations of major atmospheric pollutants such as:

- PM₂.₅ 		ii) PM₁₀		iii) SO₂			iv) NO₂

The study period covers the data 2013-2023. Data were obtained from continuous ambient air quality monitoring stations. We can’t collect the data of 2024,2025 because of ‘absence of average data’.

    - Calculation of Air Quality Index (AQI)

The Air Quality Index (AQI) was calculated using the standard methodology prescribed by the Central Pollution Control Board under the National Air Quality Index framework.

AQI Formula:

For each pollutant, a sub-index (Iₚ) was calculated using:

Ip=(IHI-ILO)(BHI-BLO)*CP-BLO+ILO

Where:

- ( CP) = Observed pollutant concentration

- (BHI), (BLO) = Breakpoint concentrations

- (IHI), (ILO) = Corresponding AQI breakpoints

The overall AQI was determined by taking the maximum sub-index value among all pollutants.

Table2. AQI level Table

    - Statistical Analysis Using SPSS

All statistical analyses were performed using SPSS (Statistical Package for the Social Sciences) software.

(a) Descriptive Statistics

Descriptive statistical measures such as:

- Mean			ii)  Standard Deviation

were calculated to understand the central tendency and dispersion of pollutant concentrations.

(b) Coefficient of Variation (CV)

The coefficient of variation was calculated to measure the relative variability of pollutants.

CV= Standard deviationMeanx100

A higher CV indicates greater variability in pollutant concentration over time.

| Coefficient of Variation (CV %) | Level of Variability | Interpretation |
| --- | --- | --- |
| CV < 10% | Very Low Variability | Data is highly consistent and very stable around the mean. |
| 10% – 20% | Low Variability | Data shows small fluctuations and is fairly reliable. |
| 20% – 30% | Moderate Variability | Data has noticeable variation but still acceptable for analysis. |
| CV > 30% | High Variability | Data is highly dispersed and shows large fluctuations around the mean. |

List 1- Coefficient of Variation analysis

(c) Correlation Analysis

Pearson’s correlation coefficient (r) was used to determine the relationship between:

- PM₂.₅ of different city’s with Agartala

- SO₂ and NO₂

- Individual pollutants and AQI

The correlation value ranges between -1 and +1:

List 2-Correlation Analysis

| Correlation Coefficient (r) | Interpretation |
| --- | --- |
| +0.70 to +1.00 | Strong positive relationship |
| +0.40 to +0.69 | Moderate positive relationship |
| +0.10 to +0.39 | Weak positive relationship |
| 0 | No correlation |
| –0.10 to –0.39 | Weak negative relationship |
| –0.40 to –0.69 | Moderate negative relationship |
| –0.70 to –1.00 | Strong negative relationship |

(d) Graphical Representation

Graphs were prepared for visual interpretation of pollutant trends, including:

- Line graphs of PM₂.₅, PM₁₀, SO₂, and NO₂

- Bar diagrams for monthly average concentrations

- Scatter plots for correlation analysis

These visual representations help identify seasonal variations and pollution peaks.

(e) Linear Trend Analysis

A linear trend model was applied to examine the long-term variation in pollutant concentration:

Y=a+bt

Where:

- Y = Pollutant concentration

- t = Time

- b = Trend coefficient

If b > 0 → Increasing trend
If b < 0 → Decreasing trend

(f) Forecasting

Time-series forecasting was performed using linear trend projection in SPSS to estimate future pollutant concentrations and AQI values. This helps in predicting possible future pollution scenarios.

    - Software Tools

- Microsoft Excel (for data cleaning and initial tabulation)

- SPSS (for statistical analysis and trend modeling)

- RESULTS

    - Graphical Representation

In this part of the analysis, the average values of Air Quality Index (AQI) and major atmospheric pollutants such as PM₂.₅, PM₁₀, NO₂, and SO₂ for Agartala, Delhi, and Guwahati were compared using graphical representation. The visualization was carried out with the help of statistical and spreadsheet software such as SPSS and Microsoft Excel to clearly illustrate the differences in pollutant levels among the selected cities.

- AQI

Fig 1- Air Quality Index (AQI) in Agartala, Delhi, and Guwahati

Agartala vs Delhi:
Delhi records much higher AQI values, often around 300 or more, indicating severe pollution. In contrast, Agartala generally shows lower AQI levels (80–135), reflecting comparatively better air quality, though occasional increases above 200 were observed.

Agartala vs Guwahati:
Agartala also shows lower AQI than Guwahati. While both cities saw a decline around 2020, Guwahati generally records higher pollution levels. Overall, Agartala maintains relatively better air quality, though a gradual rise in AQI is noticeable.

- PM2.5

Fig 2- PM₂.₅ level in Agartala, Delhi, and Guwahati

Agartala vs Delhi:
Delhi records much higher PM₂.₅ levels (about 145–180 µg/m³) compared to Agartala (about 48–72 µg/m³) during 2014–2023, indicating much more severe pollution in Delhi.

Agartala vs Guwahati:
Agartala generally shows higher PM₂.₅ than Guwahati for most years. However, Guwahati’s PM₂.₅ has steadily increased, and by 2023 both cities show nearly similar levels.

- PM10

Fig 3-  PM₁₀ levels  in Agartala, Delhi, and Guwahati

Agartala vs Delhi:
Delhi records much higher PM₁₀ levels (about 210–275 µg/m³) than Agartala (about 78–110 µg/m³), indicating far more severe particulate pollution.

Agartala vs Guwahati:
Agartala shows lower PM₁₀ levels than Guwahati throughout the study period. Guwahati records higher concentrations with a noticeable increase around 2019 and 2023.

- SO2

Fig 4- SO₂ level in Agartala, Delhi, and Guwahati

Agartala vs Delhi:
Agartala records lower and more stable SO₂ levels than Delhi. While Delhi shows higher values initially, they gradually decline over time.

Agartala vs Guwahati:
Agartala generally shows slightly lower SO₂ levels than Guwahati. Although both cities saw a drop around 2020, Guwahati shows a gradual increase and higher values in later years.

- NO2

Fig 5-    NO2   level in Agartala, Delhi, and Guwahati

Agartala vs Delhi:
The NO₂ graph indicates that Delhi consistently records much higher NO₂ concentrations than Agartala during 2014–2023. Delhi’s values generally remain between about 38 and 52 µg/m³, reflecting stronger emissions from sources such as vehicular traffic and urban activities. In contrast, Agartala maintains much lower NO₂ levels, mostly ranging between about 15 and 24 µg/m³, although a gradual increase is observed toward the later years.

Agartala vs Guwahati:
When compared with Guwahati, Agartala shows consistently lower NO₂ concentrations throughout the study period. Guwahati’s values increase steadily from around 20 µg/m³ in 2014 to about 33 µg/m³ in 2023, indicating higher pollution levels than Agartala. Although both cities experience a noticeable decline around 2020, Agartala remains comparatively lower in NO₂ concentration than Guwahati in most years.

    - Statistical Analysis

- Descriptive Statistics

| Guwahati |  |  |  |
| --- | --- | --- | --- |
| Average PM2.5 | 38.6 | SD PM2.5 | 11.4717 |
| Average PM10 | 159.7 | SD PM10 | 29.93716 |
| Average SO2 | 8.6 | SD SO2 | 1.577621 |
| Average NO2 | 26.4 | SD NO2 | 4.501851 |
| Average AQI | 135.6 | SD AQI | 29.94885 |
| Coefficient of Variation PM2.5 | 29.71944 |  |  |
| CV PM10 | 18.74587 |  |  |
| CV SO2 | 18.34443 |  |  |
| CV NO2 | 17.05247 |  |  |
| CV AQI | 22.08617 |  |  |
| CV in Percentage |  |  |  |

| Agartala |  |  |  |
| --- | --- | --- | --- |
| AV PM2.5 | 62.7 | SD PM2.5 | 7.717944 |
| AV PM10 | 93.4 | SD PM10 | 10.29779 |
| AV SO2 | 7.1 | SD SO2 | 1.197219 |
| AV NO2 | 18.7 | SD NO2 | 3.335 |
| AV AQI | 111.9 | SD AQI | 18.32697 |
| CV PM2.5 | 12.30932 |  |  |
| CV PM10 | 11.02547 |  |  |
| CV SO2 | 16.86224 |  |  |
| CV NO2 | 17.83422 |  |  |
| CV AQI | 16.37799 |  |  |
| CV in Percentage |  |  |  |

| Delhi |  |  |  |
| --- | --- | --- | --- |
| AV PM2.5 | 164.4 | SD PM2.5 | 11.6065116 |
| AV PM10 | 244.8 | SD PM10 | 24.5709856 |
| AV SO2 | 9.5 | SD SO2 | 1.43372088 |
| AV NO2 | 45.7 | SD NO2 | 4.57165178 |
| AV AQI | 305.5 | SD AQI | 21.1305466 |
| CV PM2.5 | 7.05992189 |  |  |
| CV PM10 | 10.0371673 |  |  |
| CV SO2 | 15.0917987 |  |  |
| CV NO2 | 10.0036144 |  |  |
| CV AQI | 6.9167092 |  |  |
| CV in Percentage |  |  |  |

List 3- Descriptive Statistics Analysis

The coefficient of variation (CV) is an important statistical measure used to understand the level of variability in relation to the mean of a dataset. It is calculated by dividing the standard deviation by the mean and multiplying the result by 100. Unlike standard deviation alone, CV helps compare the relative variability of pollutants across different cities even when their average concentrations differ. In this study, CV is used to examine the stability of pollution levels in Agartala, Delhi, and Guwahati.

The analysis shows that Delhi generally has lower CV values, indicating relatively stable pollutant concentrations over time. For example, the CV of PM₂.₅ and AQI is around 7%, suggesting very low variability and consistently high pollution levels. PM₁₀ and NO₂ show moderate variability with CV values around 10%, while SO₂ (about 15%) shows comparatively higher fluctuations.

In contrast, Agartala shows higher CV values, indicating greater fluctuations in pollutant concentrations. Although the overall pollution level is lower than Delhi, variations occur due to seasonal meteorological factors such as rainfall, wind speed, and regional air movement.

Guwahati represents an intermediate situation, with moderate CV values. Urban growth, vehicular emissions, and its valley-like topography surrounded by hills influence the dispersion and accumulation of pollutants.

Overall, the CV analysis shows that Delhi has consistently high but stable pollution levels, while Agartala and Guwahati experience more variable pollution patterns influenced by environmental and seasonal factors.

    - FORCAST

Fig 6- Forecast of AQI of Agartala

- Feature forecast of AQI of Agartala

List 4- AQI FORECAST OF AGARTALA

| AQI FORECAST OF AGARTALA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | AQI | Year | AQI | Forecast(AQI) |
| 2014 | 82 | 2023 | 138 | 138 |
| 2015 | 91 | 2024 |  | 138 |
| 2016 | 104 | 2025 |  | 143 |
| 2017 | 112 | 2026 |  | 148 |
| 2018 | 118 | 2027 |  | 153 |
| 2019 | 125 | 2028 |  | 158 |
| 2020 | 96 | 2029 |  | 164 |
| 2021 | 121 | 2030 |  | 169 |
| 2022 | 132 | 2031 |  | 174 |

- Feature forecast of PM2.5 of Agartala

Fig 7-   Forecast of PM2.5 of Agartala

List -5- PM2.5 FORECAST OF AGARTALA

| PM2.5 FORECAST OF AGARTALA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | PM2.5 | Year | PM2.5 | Forecast(PM2.5) |
| 2014 | 48 | 2023 | 65 | 65 |
| 2015 | 52 | 2024 |  | 66.93333333 |
| 2016 | 59 | 2025 |  | 68.86666667 |
| 2017 | 63 | 2026 |  | 70.8 |
| 2018 | 66 | 2027 |  | 72.73333333 |
| 2019 | 68 | 2028 |  | 74.66666667 |
| 2020 | 73 | 2029 |  | 76.6 |
| 2021 | 69 | 2030 |  | 78.53333333 |
| 2022 | 64 | 2031 |  | 80.46666667 |

    - Correlation Analysis

| Correlation Analysis |  |  |
| --- | --- | --- |
|  | Delhi | Guwahati |
| PM2.5 | 0.610514 | 0.480396 |
| PM10 | 0.905833 | 0.899668 |
| SO2 | -0.22656 | 0.964773 |
| NO2 | 0.831523 | 0.970967 |
| AQI | 0.413591 | 0.973229 |

list 6- 	Correlation Analysis showing pollutants in Agartala with Delhi and Guwahati

The table shows the correlation analysis of atmospheric pollutants in Agartala with Delhi and Guwahati. The correlation coefficient (r) indicates the strength of the relationship between pollutant levels in these cities.

When compared with Delhi, PM₂.₅ (r = 0.61) shows a moderate positive correlation, while PM₁₀ (r = 0.91) and NO₂ (r = 0.83) show strong positive correlations, indicating similar trends between the two cities. However, SO₂ (r = −0.23) shows a weak negative correlation, suggesting different variation patterns. The AQI (r = 0.41) indicates only a moderate relationship.

In comparison with Guwahati, most pollutants show very strong positive correlations, including PM₁₀ (r = 0.90), SO₂ (r = 0.96), NO₂ (r = 0.97), and AQI (r = 0.97). PM₂.₅ (r = 0.48) shows a moderate correlation.

Overall, the results indicate that Agartala’s pollution pattern is more closely related to Guwahati than to Delhi, particularly for PM₁₀, SO₂, NO₂, and AQI.

- Resent Scenario

| Date | Colour | AQI | Major Pollutant | Date | Colour | AQI | Major Pollutant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Feb-26 | Yellow | 115 | PM10 | 08-Mar | Yellow | 162 | PM2.5 |
| Feb-25 | Red | 356 | PM10 | 09-Mar | Yellow | 146 | PM2.5 |
| Feb-26 | Orange | 251 | PM2.5 | 10-Mar | Yellow | 191 | PM2.5 |
| Feb-27 | Orange | 263 | PM2.5 | 11-Mar | Orange | 231 | PM2.5 |
| Feb-28 | Orange | 216 | PM2.5 | 12-Mar | Yellow | 175 | PM2.5 |
| 01-Mar | Yellow | 161 | PM2.5 | 13-Mar | Yellow | 117 | PM2.5 |
| 02-Mar | Yellow | 132 | PM2.5 | 14-Mar | Light Green | 68 | PM2.5 |
| 03-Mar | Orange | 213 | PM2.5 | 15-Mar | Light Green | 66 | PM2.5 |
| 04-Mar | Orange | 252 | PM2.5 | 16-Mar | Maroon | -- |  |
| 05-Mar | Orange | 239 | PM2.5 | 17-Mar | Maroon | -- |  |
| 06-Mar | Orange | 225 | PM2.5 | 18-Mar | Yellow | 102 | PM10 |
| 07-Mar | Orange | 206 | PM2.5 | 19-Mar | Yellow | 107 | PM10 |

List 7- Present State of AQI due to Major Pollutants

  - Seasonal pollutants

Seasonal variation of atmospheric pollution in Agartala, Guwahati, and Delhi can be understood by examining how pollutant concentrations change during different seasons of the year. For this purpose, the year is generally divided into four seasons: winter (December–February), pre-monsoon or summer (March–May), monsoon (June–September), and post-monsoon (October–November). Major atmospheric pollutants such as PM₂.₅, PM₁₀, NO₂, and SO₂ are analysed according to these seasonal periods to observe how their concentrations vary throughout the year. Meteorological conditions such as temperature, rainfall, humidity, and wind speed play an important role in determining the dispersion and accumulation of pollutants. During the monsoon season, rainfall often helps remove particulate matter from the atmosphere, which usually results in comparatively lower pollution levels. In contrast, during the winter season, lower temperatures and weaker wind movement allow pollutants to remain trapped near the ground, leading to relatively higher concentration levels.

Apart from seasonal meteorological factors, human activities and cultural festivals also contribute to short-term increases in pollution levels in these cities. In Delhi, major festivals such as Diwali and Dussehra often involve the extensive use of firecrackers, which release large amounts of smoke and fine particulate matter into the air. Similarly, in Guwahati and Agartala, festivals such as Bihu, Diwali, and Lakshmi Puja may also lead to temporary increases in pollution due to the use of firecrackers and increased social activities. These festival-related emissions often cause short-term spikes in pollutants such as PM₂.₅ and PM₁₀. Therefore, the seasonal analysis of pollution across these cities should consider both meteorological influences and festival-related human activities, as together they play a significant role in shaping the overall pattern of atmospheric pollution.

  - Limitations of the Study

•	The study is mainly based on secondary data collected from official monitoring sources such as the Central Pollution Control Board (CPCB) and State Pollution Control Boards. Therefore, the analysis depends on the availability and accuracy of the recorded data.

•	The number of monitoring stations in some cities is limited, especially in Agartala smart city, which may not fully represent the overall spatial variation of pollution across the entire state.

•	The research focuses on selected atmospheric pollutants such as PM₂.₅, PM₁₀, NO₂, and SO₂, while other pollutants like O₃, CO, and NH₃ were not included due to limited data availability.

•	The study does not include detailed meteorological parameters such as wind speed, humidity, and temperature variations, which can influence pollutant dispersion and concentration.

•	Since the research compares only three cities (Agartala, Guwahati, and Delhi), the findings may not represent the complete pollution scenario of other regions in India.

•	Although seasonal and festival-related pollution patterns were discussed, detailed event-based analysis for each festival period could not be carried out due to limited high-resolution data.

- And a limitation is the data between 2024-2025 is not available on the official and unofficial sites.

•	In several cases, complete daily pollutant data were not available, and due to this limitation, it was not possible to obtain or compute the daily air quality values for all dates, which may affect the continuity of the analysis.

- Conclusion

This study analysed the air quality condition of Agartala by examining major pollutants and their statistical behaviour during the study period. The data were obtained from the Central Pollution Control Board (CPCB) and included pollutants such as PM₂.₅, PM₁₀, SO₂, NO₂, and the Air Quality Index (AQI). Various statistical techniques, including descriptive statistics, correlation analysis, coefficient of variation, graphical analysis, trend analysis, and forecasting, were used to understand pollution patterns.

The results indicate that particulate matter (PM₂.₅ and PM₁₀) plays a major role in determining AQI levels in Agartala, while gaseous pollutants remain comparatively lower. The coefficient of variation shows that pollutant concentrations fluctuate over time due to seasonal and meteorological factors. Correlation analysis suggests that some pollutants are related and may originate from common sources such as vehicular emissions and urban activities.

Trend analysis indicates a gradual increase in certain pollutants, particularly particulate matter, while forecasting suggests that pollution levels may rise in the future if urban development continues without effective control measures.

Overall, although Agartala currently experiences moderate pollution compared to larger cities, continuous monitoring, effective environmental policies, and public awareness are necessary to maintain healthy air quality in the future.

- Reference

- Basumatary, M. G., Anand, S., Prasad, S., & Nagarale, V. (2022).
Spatio-temporal analysis of air quality index: A case study in Delhi and Chennai city from 2013 to 2020.
Available at: https://www.neliti.com/publications/575872/spatio-temporal-analysis-of-air-quality-index-a-case-study-in-delhi-and-chennai

- Handhayani, Teny. "An Integrated Analysis of Air Pollution and Meteorological Conditions in Jakarta." Scientific Reports 13, 5798 (2023).

- Çelikten, Hakan. "Air Quality Index Calculation and Analysis of Pollutant Impacts in Ardahan, Türkiye." Frontiers in Life Sciences and Related Technologies 6, no. 2 (2025): 97–104.

- Ahmed, Osama, Zubair Khalid, Muhammad Tahir, and Momin Uppal. "Spatiotemporal Air Quality Mapping in Urban Areas Using Sparse Sensor Data, Satellite Imagery, Meteorological Factors, and Spatial Features." arXiv preprint arXiv:2501.11270 (2025).

- Chen, Junyu, Pratik Nag, Huixia Judy Wang, and Ying Sum. "Deep Classifier Kriging for Probabilistic Spatial Prediction of Air Quality Index." arXiv preprint arXiv:2512.23474 (2025).

- Pahari, Soham, and Sandeep Ehad Kumain. "Air Quality Prediction Using LOESS-ARIMA and Multi-Scale CNN-BiLSTM with Residual-Gated Attention." arXiv preprint arXiv:2510.22818 (2025).

- Bishoi, Biswanath, Amit Prakash, and V. K. Jain. "A Comparative Study of Air Quality Index Based on Factor Analysis and US-EPA Methods for an Urban Environment." Aerosol and Air Quality Research 9, no. 1 (2009): 1–17.

- Anggraini, Tania Septi, Hitoshi Irie, Anjar Dimara Sakti, and Ketut Wikantika. "Global Air Quality Index Prediction Using Integrated Spatial Observation Data and Geographics Machine Learning." Environmental and Sustainability Indicators (2025). https://doi.org/10.1016/S2666017225000033

- Santoso, Richard, and Karto Iskandar. "Global Air Quality Index Prediction Using Machine Learning on Major Pollutants." JADS Vol-7,NO.1  (2026)

- Suman. "Air Quality Indices: A Review of Methods to Interpret Air Quality Status." Materials Today: Proceedings 34 (2021): 863–868.

- Patel, Jay S., Hirva U. Salvi, and Neha R. Patel. "Ambient Air Quality Assessment Using Air Quality Index of Delhi." Kalpa Publications in Civil Engineering Vol-1(2017)

- Kumar, Vivek, Panuganti C. S. Devara, and Vijay Kumar Soni. "Spatiotemporal Variations in Black Carbon Aerosol Concentration and Its Correlation with Meteorological Parameters and Other Pollutants in India." MAUSAM Vol-77 (2026)

- Maltare, Nilesh N., and Safvan Vahora. "Air Quality Index Prediction Using Machine Learning for Ahmedabad City." Digital Chemical Engineering 7 (2023): 100093. https://doi.org/10.1016/j.dche.2023.100093

- Ravindiran, Gokulan, Gasim Hayder, Karthick Kanagarathinam, Avinash Alagumalai, and Christian Sonne. "Air Quality Prediction by Machine Learning Models: A Predictive Study on the Indian Coastal City of Visakhapatnam." Chemosphere 338 (2023): 139518. https://doi.org/10.1016/j.chemosphere.2023.139518

- Yenkikar, Anuradha, Ved Prakash Mishra, Manish Bali, and Tabassum Ara. "Explainable Forecasting of Air Quality Index Using a Hybrid Random Forest and ARIMA Model." MethodsX (2025). https://doi.org/10.1016/j.mex.2025.103619

- Basumatary, Mijing Gwra, Subhash Anand, Sanjeevi Prasad, and Virendra Nagarale. "Spatio-Temporal Analysis of Air Quality Index: A Case Study in Delhi and Chennai City from 2013 to 2020." International Journal of Health Sciences 6, no. S5 (2022): 12332–12352. https://doi.org/10.53730/ijhs.v6nS5.11380

- Natarajan, Suresh Kumar, Prakash Shanmurthy, Daniel Arockiam, Balamurugan Balusamy, and Shitharth Selvarajan. "Optimized Machine Learning Model for Air Quality Index Prediction in Major Cities in India." Scientific Reports 14, 6795 (2024). https://doi.org/10.1038/s41598-024-54807-1

- Guha, Anirban, Barin Kumar De, Pranab Dhar, Trisanu Banik, Monti Chakraborty, Rakesh Roy, Abhijit Choudhury, Mukunda M. Gogoi, S. Suresh Babu, and K. Krishna Moorthy. "Seasonal Characteristics of Aerosol Black Carbon in Relation to Long-Range Transport over Tripura in Northeast India." Aerosol and Air Quality Research 15, no. 3 (2015): 786–798. https://doi.org/10.4209/aaqr.2014.02.0029

- Roychowdhury, Anumita, Avikal Somvanshi, and Sharanjeet Kaur. "Winter Air Pollution in the States of Northeast India." Centre for Science and Environment (CSE), New Delhi, December 17, 2021. Technical Report.

- Shubham, Kumar, T. Gopikrishnan, and Anshuman Singh. "Forecasting and Assessment of Air Quality Dynamics in Northeast India Using Machine Learning Models." The Philippine Agricultural Scientist 108, no. 1 (2025): Article 4.

- Mathur, Vansh, Shalavya Agrawal, Naina Agrawal, Udit Pandey, and Parth Kedawat. "Comparative Analysis of Air Quality Indices Across Five Eastern States of India Using ML Models and Pollutant-Based Insights." International Journal of Distributed Computing and Technology 11, no. 1 (2025). https://doi.org/10.37628/ijdct.v11i01.13141

- Roy, B. C., S. Chakraborty, A. Roy, and P. Dhar. "Air Pollution Report of Agartala City, Tripura, India." Journal of Environmental Pollution and Control Vol -1 (2018)

- Fundamentals of Mathematical Statistics.
Gupta, S. C., & Kapoor, V. K. (2014). Fundamentals of Mathematical Statistics. Sultan Chand & Sons, New Delhi.

- Fundamentals of Applied Statistics.
Gupta, S. C., & Kapoor, V. K. (2014). Fundamentals of Applied Statistics. Sultan Chand & Sons, New Delhi.

- Central Pollution Control Board (CPCB). National Air Quality Monitoring Programme (NAMP) Data.
Available at: https://cpcb.nic.in/namp-data/

- Tripura State Pollution Control Board (TSPCB). Ambient Air Quality Monitoring Data.
Available at: https://tspcb.tripura.gov.in

- Assam Pollution Control Board (APCB). Air Quality Monitoring and Environmental Data.
Available at: https://pcbassam.org