Post-Remediation Soil Recovery in K-Dere, Ogoniland: Implications for Sustainable Environmental Restoration

Abstract

This study investigated the post-remediation effects on soil health in K-Dere, Ogoniland, Rivers State, to determine whether remediation activities have effectively restored soil physicochemical and biological functionality in a hydrocarbon-impacted environment. Composite soil samples were collected from remediated soil (RS) and unremediated soil (URS) at a depth of 0–15 cm, and laboratory analyses were conducted in triplicate using standard soil analytical methods to assess microbial biomass, enzyme activities, organic matter status, nutrient composition and key physicochemical properties. The results showed that total organic carbon decreased from 2.10% in URS to 0.55% in RS (74.0% reduction), whereas total organic nitrogen increased from 0.20% to 0.40% (100% increase). Sulphate concentration increased from 56.98 to 66.30 mg/kg (16.4% increase), nitrate from 4.6 to 10.7 mg/kg (132.6% increase), and phosphate from 5.88 to 23.4 mg/kg (298.0% increase). Bulk density declined from 7.59 to 5.24 g/cm³ (31.0% reduction), water retention capacity increased from 0.375 to 0.433 (15.5% increase), and cation exchange capacity rose from 5.1 to 18.5 cmol/kg (262.7% increase). In contrast, biological indicators declined markedly, as bacterial counts decreased from 4.0 × 10⁶ to 3.9 × 10⁵ cfu/g (90.3% reduction), fungal counts from 2.6 × 10⁶ to 3.0 × 10⁵ cfu/g (88.5% reduction), and dehydrogenase, urease and phosphatase activities decreased by 73.1%, 74.6% and 70.0%, respectively. The overall soil health measured by the Soil Quality Index (SQI) indicated better soil quality in the unremediated soil (SQI = 0.79 > 0.70) than in the remediated soil (SQI = 0.64 < 0.70). The study concludes that remediation activities in K-Dere have substantially improved soil nutrient availability and physicochemical conditions but have not yet achieved full biological recovery. It is therefore recommended that future remediation programmes integrate biological enhancement strategies, including organic amendments and microbial stimulation, supported by long-term post-remediation monitoring to ensure sustainable restoration of soil health.

Keywords: Soil, Remediation, physicochemical, Soil quality index, Microbial biomass, Soil health

1.0      Introduction

Soil is a fundamental component of terrestrial ecosystems, underpinning agricultural productivity, biodiversity conservation and human livelihoods (Wonodi & Ekpete, 2021; Gorai et al., 2021). In oil-producing regions, however, prolonged industrial activities often lead to severe soil degradation through hydrocarbon pollution and associated heavy metal contamination (Omokaro, 2024). The Niger Delta region of Nigeria, particularly Ogoniland, represents one of the most environmentally impacted oil-producing areas globally, where decades of oil spills, gas flaring and pipeline failures have compromised soil quality and ecosystem integrity (Nwokoro & Chima, 2018).

K-Dere community in Gokana Local Government Area of Rivers State lies at the core of Ogoniland and has experienced extensive environmental degradation due to petroleum exploration and production. The United Nations Environment Programme (UNEP) environmental assessment of Ogoniland in 2011 documented widespread soil contamination and recommended urgent remediation (UNEP, 2011). In response, the Nigerian government initiated the Hydrocarbon Pollution Remediation Project (HYPREP) to clean up and restore polluted sites, including those in K-Dere.

Despite ongoing remediation activities, concerns persist regarding the actual recovery of soil health in remediated areas. According to Nwokoro and Chima (2017), local farmers continue to report low crop yields, while scientific, site-specific post-remediation data remain limited. Most available assessments are policy-driven and lack detailed evaluations of the soil physicochemical and biological indicators required to confirm ecological recovery. This study therefore aims to investigate the impact of post-remediation activities on soil health in K-Dere, Ogoni, Rivers State, by comparing remediated and unremediated soils. Specifically, the study evaluates microbial biomass, enzyme activities, nutrient composition, organic matter status and key physicochemical properties to determine whether remediation has restored soil functionality for sustainable agricultural and ecological use.

2.0      Materials and Methods

Study Area

The study was conducted in K-Dere community, Gokana Local Government Area of Rivers State, Nigeria, located within the lowland rainforest zone of the Niger Delta. The area experiences a humid tropical climate with bimodal rainfall, receiving approximately 2000–3000 mm annually, while mean temperatures range between 25°C and 30°C. The region supports dense rainforest vegetation and is traditionally used for subsistence agriculture (Agboola & Nyiayaana, 2024). Two sites were selected: a remediated site previously impacted by hydrocarbon pollution and treated under a formal remediation project, and an unimpacted control site with no documented history of oil contamination. The unimpacted site is located at 4.659969° N, 7.270142° E, while the remediated site lies at 4.663436° N, 7.283530° E.

Map  1: Map showing the sampling points from two sampling locations in K-Dere, Ogoniland.

Sample Collection and Preservation

A descriptive and analytical research design was adopted to enable direct comparison of soil quality indicators between remediated and unremediated soils. Composite soil samples were collected from two locations designated as Location A (remediated soil) and Location B (unremediated soil). At each location, ten subsamples were randomly collected using a soil auger at a depth of 0–15 cm and composited to obtain representative samples. Sampling was carried out on 4 September 2025.

Samples were placed in labelled polyethylene bags, preserved in ice boxes, and transported to the laboratory. In the laboratory, soils were air-dried under ambient conditions in a well-ventilated environment away from direct sunlight, gently crushed, sieved through a 2 mm mesh and homogenised in accordance with standard soil preparation procedures (International Organization for Standardization [ISO], 2006).

Sample Analysis

Laboratory analyses were conducted in triplicate and covered microbial biomass, enzyme activities, nutrient composition, organic matter content and physicochemical properties. Total heterotrophic bacteria and fungi were enumerated using standard culture techniques on nutrient media, and results were expressed as colony-forming units per gram of soil (cfu/g) following established soil microbiological protocols (ISO, 2013). Dehydrogenase activity was determined using the triphenyl tetrazolium chloride reduction method, urease activity by ammonium release from urea, and phosphatase activity using p-nitrophenyl phosphate as a substrate, following standard soil enzymology methods as described by Wang et al. (2023).

Total organic carbon was determined using the Walkley–Black method, while total organic nitrogen was analysed using the Kjeldahl method in accordance with Association of Official Analytical Chemists (AOAC, 2019) procedures. Phosphate was determined by colourimetric spectrophotometry, nitrate by the brucine method, and sulphate by titrimetric analysis, following standard environmental analytical methods (ASTM International, 2017; ASTM International, 2009). Bulk density was measured using the core method (ASTM, 2011), water retention capacity by saturation and drainage techniques, soil texture by the ribbon method (Food and Agriculture Organization of the United Nations, 2006), and cation exchange capacity using standard soil chemical procedures (ISO, 2023).

Assessment of Soil Quality Index

The determination of overall soil health to support agricultural productivity commonly practised in the area used a standardised model (Doran & Parkin, 1994; Andrews et al., 2004). The approach used a total of 15 soil quality indicators (physical, chemical and biological), described as the minimum data set (MDS), as recommended by the United States Department of Agriculture, Natural Resources Conservation Service (USDA-NRCS, 2001, 2015a, 2015b).

Scoring of the MDS was modelled according to Kahsay et al. (2025), Doran and Parkin (1994), and Andrews et al. (2004), using linear normalisation in which the best-performing value in each pair was assigned a score of 1.0 and the other value was scored relative to it. Indicators were categorised as “more-is-better”, “less-is-better” or “optimum-range”, following the Soil Management Assessment Framework (Andrews et al., 2004). Equal weighting was assigned to the indicators to avoid disparity in the contributions of individual soil quality indicators to overall soil health.

Egn (1)

Where Wi = Weight of the ith indicator and n =total number of soil quality indicators (MDS)

The soil quality index was evaluated using Egn (2)  as:

Egn (2)

Data Analysis

Data obtained were subjected to descriptive statistical analysis, and mean values were used to compare remediated and unremediated soils.

3.0    Results

Table 1: Microbial biomass, enzyme activities, and biodiversity of soils from K-Dere

| Parameter | URS | RS |
| --- | --- | --- |
| Bacteria (cfu/g) | 4.0 × 10⁶ | 3.9 × 10⁵ |
| Fungi (cfu/g) | 2.6 × 10⁶ | 3.0 × 10⁵ |
| Dehydrogenase (µg TPF g⁻¹ h⁻¹) | 2.6 | 0.7 |
| Urease (µg NH₄⁺ g⁻¹ h⁻¹) | 28.0 | 7.1 |
| Phosphatase (µg pNP g⁻¹ h⁻¹) | 55.0 | 16.5 |
| Bacterial species | Pseudomonas, Staphylococcus, Klebsiella | Bacillus, Staphylococcus, Klebsiella |
| Fungal species | Aspergillus, Fusarium, Saccharomyces, Penicillium | Aspergillus, Fusarium |

Note: URS = Unremediated Soil Sample, RS = Remediated Soil Sample

Table 2: Nutrient and physicochemical properties of soils from K-Dere

| Parameter | URS | RS |
| --- | --- | --- |
| TOC (%) | 2.10 | 0.55 |
| TON (%) | 0.20 | 0.40 |
| SO₄²⁻ (mg/kg) | 56.98 | 66.30 |
| NO₃⁻ (mg/kg) | 4.6 | 10.7 |
| PO₄³⁻ (mg/kg) | 5.88 | 23.4 |
| Soil texture | Loam | Sandy loam |
| Bulk density (g/cm³) | 7.59 | 5.24 |
| Water retention capacity | 0.375 | 0.433 |
| Soil compaction | 12.1 | 45.8 |
| CEC (cmol/kg) | 5.1 | 18.5 |

Note: URS = Unremediated Soil Sample, RS = Remediated Soil Sample

Figure 1: concentration of microbial biomass activities in URS and RS sample from K-Dere, Ogoni

Figure 2: concentration of enzymes activities in URS and RS sample from K-Dere, Ogoni

Figure 3: Concentrations of TON and TOC in RS and URS samples from K-Dere, Ogoni.

Figure 4: Concentrations of nutrient composition in RS and URS samples from K-Dere, Ogoni.

Figure 5: concentration of Physiochemical Properties in URS and RS sample from K-Dere,   Ogoni

| Table 3. Results from the evaluation of Soil Quality Index using scoring functions (Doran & Parkin, 1994; Andrews et al., 2004 ; USDA-NRCS, 2001). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Quality Indicator (MDS) | Scoring rule | RS (Xi) | URS (Yi) | RS Score (Sx) | URS Score (Sy) | Weight (Wi) | Weighted Score (W-RS) | Weighted Score (W-URS) |
| Bacteria | More is better | 3.9 × 10⁵ | 4.0 × 10⁶ | 0.098 | 1 | 0.066 | 0.0066 | 0.0666 |
| Fungi | More is better | 3.0 × 10⁵ | 2.6 × 10⁶ | 0.115 | 1 | 0.066 | 0.0077 | 0.0666 |
| Dehydrogenase | More is better | 0.7 | 2.6 | 0.269 | 1 | 0.066 | 0.0179 | 0.0666 |
| Urease | More is better | 7.1 | 28 | 0.254 | 1 | 0.066 | 0.0169 | 0.0666 |
| Phosphatase | More is better | 16.5 | 55 | 0.3 | 1 | 0.066 | 0.0199 | 0.0666 |
| pH | Optimum | 6.23 | 6.21 | 1 | 1 | 0.066 | 0.0666 | 0.0666 |
| TOC | More is better | 0.55 | 2.1 | 0.262 | 1 | 0.066 | 0.0174 | 0.0666 |
| TON | More is better | 0.4 | 0.2 | 1 | 0.5 | 0.066 | 0.0666 | 0.0333 |
| SO₄² | More is better | 66.3 | 56.98 | 1 | 0.859 | 0.066 | 0.0666 | 0.0572 |
| NO₃⁻ | More is better | 10.7 | 4.6 | 1 | 0.43 | 0.066 | 0.0666 | 0.0286 |
| PO₄³ | More is better | 23.4 | 5.88 | 1 | 0.251 | 0.066 | 0.0666 | 0.0167 |
| Bulk Density | Less is better | 5.24 | 7.59 | 1 | 0.69 | 0.066 | 0.0666 | 0.0459 |
| Water Retention | More is better | 0.433 | 0.375 | 1 | 0.866 | 0.066 | 0.0666 | 0.0577 |
| Soil Compaction | Less is better | 45.8 | 12.1 | 0.264 | 1 | 0.066 | 0.0176 | 0.0666 |
| CEC | More is better | 18.5 | 5.1 | 1 | 0.276 | 0.066 | 0.0666 | 0.0184 |
| TOTAL SQI |  |  |  |  |  |  | 0.6368 | 0.7906 |

Figure 6: concentration of Soil Quality Index using scoring functions activities in URS and RS sample from K-Dere, Ogoni

4.0      Discussion

Physicochemical properties of RS and URS

The post-remediation assessment of soils from K-Dere revealed clear numerical variations between remediated soil (RS) and unremediated soil (URS), as presented in Tables 1, 2 and 3, indicating that remediation activities significantly influenced soil chemical, physical and biological properties. From the results in Table 2, total organic carbon (TOC) declined from 2.10% in the unremediated soil (URS) to 0.55% in the remediated soil (RS) (74.0% reduction). This level of TOC reduction is comparable to the 71–78% decreases reported by Bessong et al. (2023) and Lee et al. (2020) following active remediation of hydrocarbon-impacted soils using organic amendments. Similar reductions were also reported by Michael-Igolima et al. (2022), who attributed TOC decline to enhanced mineralisation and removal of petroleum-derived carbon during remediation. The observed reduction therefore indicates effective contaminant removal rather than nutrient depletion.

Total organic nitrogen (TON), however, increased from 0.20% in URS to 0.40% in RS (100% increase), as shown in Table 2. This trend is broadly consistent with findings by Lee et al. (2020) and Elgazali et al. (2023), who reported nitrogen increases ranging from 60% to 120% following soil remediation, attributed to microbial nitrogen fixation and improved nitrogen cycling. The increase observed in this study suggests partial restoration of soil fertility despite carbon loss.

Inorganic nutrient availability also improved markedly after remediation. Sulphate concentration increased from 56.98 mg/kg in URS to 66.3 mg/kg in RS (16.4% increase), nitrate from 4.6 mg/kg to 10.7 mg/kg (132.6% increase), and phosphate from 5.88 mg/kg to 23.4 mg/kg (298.0% increase), as presented in Table 2. Comparable phosphate increases of 200% to 350% have been reported by Kumar et al. (2015) and Ch'ng et al. (2014), who linked such improvements to increased mineralisation and reduced hydrocarbon inhibition of phosphorus-solubilising microbes. The pronounced nitrate increase further suggests enhanced post-remediation nitrification processes.

Physical soil properties also reflected significant modification. Bulk density decreased from 7.59 g/cm³ in URS to 5.24 g/cm³ in RS (31.0% reduction), while water retention capacity increased from 0.375 to 0.433 (15.5% increase), as shown in Table 2. Similar reductions in bulk density (12%) were documented by Toková et al. (2020) following soil rehabilitation, attributed to improved aggregation and pore distribution. The increase in cation exchange capacity from 5.1 cmol/kg to 18.5 cmol/kg (262.7% increase) further corroborates improved nutrient retention capacity, consistent with findings by Zubairu et al. (2023), who reported CEC increases exceeding 50% in remediated Nigerian soils.

The sharp increase in soil compaction from 12.1 to 45.8 is a major detrimental physical change in the RS. This is not merely a surface effect; high compaction combined with the loss of TOC creates a cycle of reduced structural resilience. Without organic matter to provide structural resilience and without microbes to create biological pores, the soil may remain compacted, further inhibiting the biological recovery needed to improve it (USDA-NRCS, 2019). Heavy-duty machinery, such as crawler tractors used for excavations, bulldozers, tippers and similar equipment, is commonly used during remediation and could contribute significantly to the observed high soil compaction in RS at the study area. High soil compaction has been attributed to restricted root growth, earthworm activity, aeration and nutrient availability (USDA-NRCS, 2019).

Soil microbial biomass

Results presented in Table 1 indicate a decrease in total bacterial counts from 4.0 × 10⁶ cfu/g in URS to 3.9 × 10⁵ cfu/g in RS (90.3% reduction), while fungal counts declined from 2.6 × 10⁶ cfu/g to 3.0 × 10⁵ cfu/g (88.5% reduction). Studies have reported the vital functions of fungi in soil aggregation and the symbiotic acquisition of nutrients by plants (Bogati et al., 2025). However, this significant fungal population decline is known to impair soil biological resilience and its ability to recover from future environmental stresses (Melo et al., 2024).

Dehydrogenase activity reduced from 2.6 to 0.7 µg TPF g⁻¹ h⁻¹ (73.1% reduction), urease from 28.0 to 7.1 µg NH₄⁺ g⁻¹ h⁻¹ (74.6% reduction), and phosphatase from 55.0 to 16.5 µg pNP g⁻¹ h⁻¹ (70.0% reduction). These declines are consistent with reports by Lee et al. (2020) and Gospodarek et al. (2021), who observed temporary microbial suppression following aggressive remediation treatments. Because dehydrogenase occurs only in living, metabolically active cells and is not stabilised in the soil matrix like other enzymes, this parameter serves as a direct indicator of current microbial oxidative activity. The 73.1% drop in dehydrogenase activity confirms that the microbial biomass in the remediated soil (RS) is not only lower in quantity but also significantly less active.

Urease is known for converting urea into ammonia. This decrease in activity by 74.6% may be linked to the reduction in organic carbon (TOC) available as a substrate for urease-producing microbes, or to the presence of residual remediation chemicals that act as urease inhibitors (Ward Laboratories, Inc., 2019).

The activities of dehydrogenase, urease and phosphatase provide a direct measure of the soil's metabolic capacity across carbon, nitrogen and phosphorus cycles (Rana et al., 2025). The reduced microbial diversity observed in Table 1, with fewer bacterial and fungal taxa in RS compared with URS, further supports the assertion that biological recovery lags behind chemical and physical restoration. Similar biodiversity reductions have been reported by Ramadass et al. (2017), who emphasised that microbial communities require extended post-remediation periods to re-establish functional stability. Overall, the results of the present study demonstrate that remediation activities in K-Dere improved nutrient availability, soil structure, water retention capacity and cation exchange capacity. However, the simultaneous reductions in organic carbon, microbial biomass, enzyme activities and biodiversity indicate that biological restoration has not progressed at the same pace as chemical and physical recovery.

Soil Quality Index

The Soil Quality Index (SQI), based on the selected minimum data set, indicated better soil quality status for the unremediated soil (SQI = 0.79 > 0.70). This observation is largely driven by high microbial abundance and enzyme activities. The remediated soil exhibited lower overall soil quality (SQI = 0.64 < 0.70), despite improvements in physicochemical properties such as nutrient availability, cation exchange capacity and water retention. However, reduced biological indicator scores suggest delayed recovery of soil microbial functions following remediation.

5.0	Limitation of the Study

This study was conducted as a pilot investigation using a single remediated site and did not evaluate the specific remediation techniques employed at the site. Consequently, the findings may not be fully generalisable to other remediated soils within the study area or similar hydrocarbon-impacted environments. Nevertheless, the results provide valuable preliminary insights into potential post-remediation soil health outcomes and may serve as a useful basis for future, more comprehensive assessments involving multiple sites and remediation approaches.

6.0     Conclusion

The study demonstrates that remediation activities in K-Dere have significantly altered soil health indicators. Improvements in nutrient composition, bulk density, water retention capacity and cation exchange capacity suggest partial restoration of soil fertility. However, reductions in organic carbon, microbial biomass and enzyme activities indicate that biological functioning has not fully recovered.

The SQI assessment for the studied MDS confirms that the remediation process resulted in an approximately 19.5% decrease in overall soil quality. While the remediated soil (RS) shows improved nutrient concentrations and cation exchange capacity, it also shows severe biological suppression, substantial loss of organic carbon and a critical increase in soil compaction.

These findings highlight that remediation success should be evaluated beyond contaminant removal by incorporating biological and functional soil indicators to support long-term sustainability. The study recommends an integrated remediation approach that combines physicochemical clean-up with biological enhancement measures, such as organic amendments (compost, biosolids or biochar), microbial inoculation and structural remediation practices that can prevent, reduce and break up compacted layers and restore oxygen and water pathways after remediation operations. Long-term soil monitoring should be institutionalised to track post-remediation recovery trends. Regulatory agencies, including NOSDRA and NESREA, should enforce continuous environmental monitoring and ensure that remediation programmes incorporate site-specific soil management strategies tailored to local ecological conditions in K-Dere.

Declaration of AI Use

This manuscript was prepared through the combined contributions of all author(s), including contributions to the study design, data, content development, results, interpretation and related scholarly work. The author(s) acknowledge the use of Grammarly and ChatGPT to assist with grammar checking, language refinement and reference formatting. These AI-assisted tools were not used as authors and did not replace the intellectual contributions or scholarly judgement of the author(s). All AI-assisted outputs, including content, references and interpretations, were carefully reviewed, revised, verified and approved by the author(s). The author(s) accept full responsibility for the accuracy, integrity and final content of the manuscript.

References

American Public Health Association, American Water Works Association, & Water Environment Federation. (2017). Standard methods for the examination of water and wastewater (23rd ed.). https://e-wef.org/Store/ProductDetails.aspx?productId=58296758

ASTM International. (2009). Standard test methods for laboratory determination of density (unit weight) of soil specimens (ASTM D7263-09). https://doi.org/10.1520/D7263-09

ASTM International. (2017). Standard test method for anions in water by suppressed ion chromatography (ASTM D4327-17). https://doi.org/10.1520/D4327-17

Andrews, S. S., Karlen, D. L., & Cambardella, C. A. (2004). The soil management assessment framework: A quantitative soil quality evaluation method. Soil Science Society of America Journal, 68(6), 1945–1962. https://doi.org/10.2136/sssaj2004.1945

AOAC International. (2019). Official methods of analysis of AOAC International (21st ed.). https://members.aoac.org/AOAC/AOAC/Item_Detail.aspx?Category=OMA&iProductCode=1121

Bessong, A. R., Edu, E. N., Asuquo, N. E., Ivon, E. A., Inah, A. S., & Bebia, O. (2023). Enhanced microbial and total petroleum hydrocarbon degradation in crude-oil polluted soils using agro-wastes. Pollution, 9(4), 1741–1753. https://doi.org/10.22059/poll.2023.357572.1863

Bogati, K. A., Sewerniak, P., & Walczak, M. (2025). Unraveling the effect of soil moisture on microbial diversity and enzymatic activity in agricultural soils. Microorganisms, 13(6), Article 1245. https://doi.org/10.3390/microorganisms13061245

Doran, J. W., & Parkin, T. B. (1994). Defining and assessing soil quality. In J. W. Doran, D. C. Coleman, D. F. Bezdicek, & B. A. Stewart (Eds.), Defining soil quality for a sustainable environment (SSSA Special Publication No. 35, pp. 3–21). Soil Science Society of America. https://doi.org/10.2136/sssaspecpub35.c1

Elgazali, A., Althalb, H., Elmusrati, I., Ahmed, H. M., & Banat, I. M. (2023). Remediation approaches to reduce hydrocarbon contamination in petroleum-polluted soil. Microorganisms, 11(10), Article 2577. https://doi.org/10.3390/microorganisms11102577

Food and Agriculture Organization of the United Nations. (2006). Guidelines for soil description (4th ed.). https://www.fao.org/4/a0541e/a0541e.pdf

Gospodarek, J., Rusin, M., Barczyk, G., & Nadgórska-Socha, A. (2021). The effect of petroleum-derived substances and their bioremediation on soil enzymatic activity and soil invertebrates. Agronomy, 11(1), Article 80. https://doi.org/10.3390/agronomy11010080

Ch’ng, H. Y., Ahmed, O. H., & Majid, N. M. A. (2014). Improving phosphorus availability in an acid soil using organic amendments produced from agro-industrial wastes. The Scientific World Journal, 2014, Article 506356. https://doi.org/10.1155/2014/506356

International Organization for Standardization. (2023). ISO/TS 22171:2023: Soil quality—Determination of potential cation exchange capacity (CEC) and exchangeable cations buffered at pH 7, using a molar ammonium acetate solution. https://www.iso.org/standard/80891.html

International Organization for Standardization. (2006). ISO 11464:2006: Soil quality—Pretreatment of samples for physico-chemical analysis. https://www.iso.org/standard/37718.html

International Organization for Standardization. (2013). ISO 4833-1:2013: Microbiology of the food chain—Horizontal method for the enumeration of microorganisms—Part 1: Colony count at 30 °C by the pour plate technique. https://www.iso.org/standard/53728.html

Kahsay, A., Haile, M., Gebresamuel, G., & Mohammed, M. (2025). Developing soil quality indices to investigate degradation impacts of different land use types in Northern Ethiopia. Heliyon, 11, Article e41185. https://doi.org/10.1016/j.heliyon.2024.e41185

Kumar, S., Srivastava, A., & Gupta, A. (2015). Effect of organic amendments on availability of different chemical fractions of phosphorus. Agricultural Science Digest, 35(2), 83–88. https://doi.org/10.5958/0976-0547.2015.00033.6

Melo, D. M. A. de, Pérez-Marín, A. M., Araújo, A. E. de, Oliveira, M. R. G. de, Macedo, R. S., & Nascimento, S. M. S. G. do. (2024). Soil quality indicators in peasant agroecosystems in Paraíba State, Brazil. Tropical Conservation Science, 17, 1–5. https://doi.org/10.1177/19400829231215492

Michael-Igolima, U., Abbey, S. J., & Ifelebuegu, A. O. (2022). A systematic review on the effectiveness of remediation methods for oil-contaminated soils. Environmental Advances, 9, Article 100319. https://doi.org/10.1016/j.envadv.2022.100319

Nwokoro, C. V., & Chima, F. O. (2017). Impact of environmental degradation on agricultural production and poverty in rural Nigeria. American International Journal of Contemporary Research, 7(2), 1–9. https://aijcr.thebrpi.org/journals/Vol_7_No_2_June_2017/2.pdf

Nwokoro, C. V., & Chima, F. O. (2018). Evaluation of impact of unsustainable agricultural practices on rural poverty and environmental resource management in Ogoni, Nigeria. International Journal of Education and Human Developments, 4(4), 11–25.

Omokaro, G. O. (2024). Oil extraction and the environment in Nigeria’s Niger Delta: A political-industrial ecology (PIE) perspective. Asian Journal of Advanced Research and Reports, 18(11), 161–169. https://doi.org/10.9734/ajarr/2024/v18i11784

Ramadass, K., Megharaj, M., Venkateswarlu, K., & Naidu, R. (2017). Ecotoxicity of measured concentrations of soil-applied diesel: Effects on earthworm survival, dehydrogenase, urease and nitrification activities. Applied Soil Ecology, 119, 1–7. https://doi.org/10.1016/j.apsoil.2017.05.017

Rana, S., Swain, D. K., Bhattacharya, P., Roy, A., Samanta, S., Seleiman, M. F., Ali, N., & Dey, P. (2025). Soil biochemical quality indices can capture transitional changes of tillage and residue regime in lateritic soils. BMC Plant Biology, 25, Article 1128. https://doi.org/10.1186/s12870-025-07212-3

Lee, S. H., Lee, J. H., Jung, W. C., Park, M., Kim, M. S., Lee, S. J., & Park, H. (2020). Changes in soil health with remediation of petroleum hydrocarbon contaminated soils using two different remediation technologies. Sustainability, 12(23), Article 10078. https://doi.org/10.3390/su122310078

Agboola, S. T., & Nyiayaana, K. (2024). Oil exploitation and conflict in Kegbara Dere community in Rivers State, Nigeria. American Journal of Social and Humanitarian Research, 5(3), 73–85. https://doi.org/10.31150/ajshr.v5i3.2733

Gorai, T., Yadav, P. K., & Saha, R. (2021). Conservation of soil biodiversity for agricultural productivity and sustainability. Agriculture Letters, 2(1), 24–25.

Toková, L., Igaz, D., Horák, J., & Aydin, E. (2020). Effect of biochar application and re-application on soil bulk density, porosity, saturated hydraulic conductivity, water content and soil water availability in a silty loam Haplic Luvisol. Agronomy, 10(7), Article 1005. https://doi.org/10.3390/agronomy10071005

United Nations Environment Programme. (2011). Environmental assessment of Ogoniland. https://www.unep.org/resources/report/environmental-assessment-ogoniland

United States Department of Agriculture, Natural Resources Conservation Service. (2001). Guidelines for soil quality assessment in conservation planning. https://www.nrcs.usda.gov/sites/default/files/2023-04/nrcs142p2_050963.pdf

United States Department of Agriculture, Natural Resources Conservation Service. (2015a). Soil quality indicators: Physical, chemical, and biological indicators for soil quality assessment and management. https://www.nrcs.usda.gov/sites/default/files/2022-10/indicator_sheet_guide_sheet.pdf

United States Department of Agriculture, Natural Resources Conservation Service. (2015b). Soil quality indicators: Chemical indicators and soil functions. https://www.nrcs.usda.gov/sites/default/files/2022-10/chemical_indicators_overview.pdf

United States Department of Agriculture, Natural Resources Conservation Service. (2019). Soil health—Bulk density, moisture, and aeration: Guide for educators. https://www.nrcs.usda.gov/sites/default/files/2022-10/Soil%20Bulk%20Density%20Moisture%20Aeration.pdf

Wang, S., Wu, M., Zhong, S., Sun, J., Mao, X., Qiu, N., & Zhou, F. (2023). A rapid and quantitative method for determining seed viability using 2,3,5-triphenyl tetrazolium chloride (TTC): With the example of wheat seed. Molecules, 28(19), Article 6828. https://doi.org/10.3390/molecules28196828

Ward Laboratories, Inc. (2019). Soil enzyme interpretation guide (Rev. 1.0).

Wonodi, A., & Ekpete, O. A. (2021). Impact of cement production factories on the physical and chemical characteristics of soil within production areas in Port Harcourt metropolis, Rivers State, Niger Delta, Nigeria. Faculty of Natural and Applied Sciences Journal of Scientific Innovations, 3(1), 50–59. https://fnasjournals.com/index.php/FNAS-JSI/article/view/21

Zubairu, A. M., Michéli, E., Ocansey, C. M., Boros, N., Rétháti, G., Lehoczky, É., & Gulyás, M. (2023). Biochar improves soil fertility and crop performance: A case study of Nigeria. Soil Systems, 7(4), Article 105. https://doi.org/10.3390/soilsystems7040105