EVALAUATION OF THE BIODEGRADATION POTENTIAL OF SURFACTANTS FROM COMMERCIAL LIQUID DETERGENTS

Abstract

Biodegradation is an important factor for reduction and removal of organic contaminants from the environment and the evaluation of biodegradability of anthropogenic organic substances is an essential parameter for environmental risk assessment. The biodegradability of surfactants in seven common liquid detergents (Morning fresh, Mama lemon, Newday fresh, LB wash ‘N’Wax, Classic Handwash Fresh, Lily fresh and  Standard Sodium Dodecysulphate) were evaluated. Standard microbiological methods were used to isolate and identify the total heterotrophic bacterial count present in water (tap and river water) samples. The biodegradability of surfactants was evaluated using the river die-away method. Total viable count, changes in pH, nitrates, sulphates and Chemical Oxygen Demand were also assessed. The total viable heterotrophic bacterial count of the tap and river water was recorded as 3.4 x 105 and 1.57 x 10s respectively while the mean counts of detergent utilizing bacteria recorded in percentages ranged from 0.1 t0 0.9 and 1.9 – 5.2 in tap and river water respectively with SDS recording the lowest and LB wash ‘N’wax recording the highest value. Escherichia coli, Pseudomonas aeruginosa, Citrobacter freundii, Staphylococcus haemolyticus , Enterobacter aerogenes, Flavobacterium sp., Bacillus sp. and Salmonella sp. were found to effect degradation with Pseudomonas aerogenosa being the predominant organism. The initial and final mean pH of the river water in the presence of the detergents over a 28-day period of degradation ranged from 7.20-7.72, 7.50-7.60, 7.30-8.60, 6.80-8.64, 6.70-8.70, 6.75-8.25, 6.52-8.50 in LB wash’N’wax, Classic hard wash, Morning Fresh, Mama lemon, New day fresh and Lily fresh respectively. Changes in the total viable count during the Water Die-Away Test (Cfu/ml) ranged from initial to final values of 1.0 x 104 -7.6 x 104, 1.34 x 104 -7.1x 104, 1.2 x 104 - 9.4 x 104, 1.3 x 104 - 7.8 x 104, 1.5 x 104 - 9.8 x 104, 1.6 x 104 -1.26 x 104 and 1.4 x 104 – 6.0 x 104. The changes in chemical oxygen demand (COD), sulphate, nitrate and surfactant (%) concentrations of the detergents during the River-water Die-away test ranged from initial and final values of  100-250 and 20-25; 45-75 and 5-28; 10-30 and 1.80 and 3.21; 100-100 and 10-50 respectively. There were significant differences (P<0.05) between the mean pH and nitrate values; mean pH and sulphate values; and, mean pH and COD values of the samples respectively during the biodegradation test. With the exception of SDS, it was concluded that the detergent that supported the highest microbial growth was the most easily degraded and have provided an insight to the fate of these surfactant components in the aquatic environment.

Key words: Biodegradation, Surfactants, Liquid detergents

Introduction

Biodegradation is the complex microbial breakdown or mineralization of complex materials into simple inorganic constituents such as carbon dioxide, water and mineral components (Okpokwasili and Mbipom, 2004). The natural decay and recycling of materials by a wide range of life forms including microorganisms is termed biodegradation and is perceived as a beneficial process (Ogu et al., 2016). The natural environment is predominantly aerobic, which for a long time has led to a focus on the biodegradation behavior of chemicals under aerobic conditions. The efficiency of microbial degradation is often seriously impeded by two major factors namely poor accessibility of lipophilic compounds to the microbes and toxicity effect which the substrate exert on the microbes (Schwatz and Bar, 1995).

Surfactants belong to a group of chemicals of high environmental relevance due to their large production volumes. They are mainly discharged into the environment by the wastewater pathway, either after treatment in a wastewater treatment plant or directly where no treatment system is available. Environmental compartments which may be influenced by surfactants are the freshwater environment (water body and sediment), the soil if surfactant-loaded sewage is added, and the marine environment.

Surfactants can be described with the tail-head model: the tail symbolizes the hydrophobic group, and the head the hydrophilic group. As a result of the fact that detergent surfactants are mainly used in aqueous systems, surfactants are classified by the chemical structure of the hydrophilic group which is present after dissociation in aqueous solution. Different surfactant classes include anionic, non-ionic, cationic and amphoteric surfactants (Merrettig-Bruns and Jelen, 2009). These molecules have regions that can either attract or repel water, making them effective in reducing surface tension  and enhancing the solubility and emulsification of substances that do not dissolve in water (Ohalete et al., 2024).

Anionic surfactants are contained in large amount in most detergents. These surface-active compounds show anionic groups in their hydrophilic part and have a small counter ion; example is sodium or potassium, which has only minor influence on the properties of the substance. The most important anionic surfactants used in laundry detergents are soaps, alkyl benzene sulfonates, alkyl sulfates, alkylethersulfates and secondary alkane sulfonates.

Non-ionic surfactants share in overall surfactant production and use has been increasing steadily since the 1970’s. The major contributors to this increase have been ethoxylates of fatty alcohols, oxo alcohols and secondary alcohols obtained by reaction of the corresponding alcohols with ethylene oxide. The higher use of non ionic surfactants in detergent formulations has partly been concomitant with the trend to wash at lower temperatures and with changes in the production shares of different fibers. The most important non ionic surfactants are alcohol ethoxylates, alkyl phenol ethoxylates, fatty acid alkanolamides, alkylamine oxides, N-methylglucamides and alkyl polyglycosides (Kosswig, 2009).

Cationic surfactants in detergent formulations are used as fabric softener in washing processes. The most important ones are quaternary nitrogen compounds-mono or di- alkyl quaternary compounds, esterified mono or di-alkyl quaternary compounds (esterquats) and imidazole derivatives. Esterquats possess favourable ecotoxicological and toxicological properties. Alkylated imidazole derivatives are also used as fabric softeners, and due to dermatological compatibility, as body care products (Merrettig-Bruns and Jelen, 2009).

Detergents which are substances or preparations containing soaps or other surfactants intended for water based laundry or dishwashing processes. Detergents may be (liquid, powder, paste, bar, etc), widely used in any form for household laundry products, domestic and industrial cleaners, cosmetics, and industrial purposes. The most modern detergent formulation contain up to one-third of the surfactant, larger amounts of perfumes, colouring agents, whiteners, enzymes and other components (Smulders et al., 2009).

The widespread use of surfactants and their release into the environment has resulted in an upsurge of interest in the biodegradability of these compounds. The need for biodegradability studies comes to a great extent as a result of consumer use and disposal patterns of detergent chemicals prior to discharge into rivers and estuaries (Okpokwasili and Mbipom, 2004).

In most developing countries like Nigeria, environmental problems are severe and very challenging. The quality of human life and indeed the survival of many species are threatened by the presence of industrial chemicals in the environment. However, microorganisms in natural environments such as soils and rivers make major contributions to the natural attenuation of pollutants..

MATERIALS AND METHODS

Sources of Samples

The liquid detergents (LB wash. ‘N’ wax , Newday Fresh, Morning Fresh, Mama Lemon, Classic Hand Wash and Lily Fresh ) were purchased from the three major markets in Onitaha metropolis, Anambra State, Nigeria. Sodium Dodecyl Sulphate (SDS) used as standard was obtained from Sigma Chemical Company, St Louis, Missouri, USA. The river water sample used was collected from River Niger located at Onitsha, Anambra State and the tap water sample was taken from the Quality Control Microbiology Laboratory of Kingsize Pharmaceuticals Nigeria Limited, Ogidi, Anambra State.

Microbiological Analysis

The isolation of pure culture from the water samples were carried out by plating out 0.1ml of dilutions 10-2 to 10-4 for each sample using the pour plate technique on Nutrient agar and the   total aerobic count was determined.(Chesbrough, 2006)

Detergent-utilizing bacterial count

The bacteria from the river and tap water samples that could grow on agar medium ( Kcl 0.6g, MgS04.7H20 7.0g, Nacl 5.0g, NH4N03 1.0g, Detergent 10ml, Agar 20g, Deionized water 1 ml) containing a test liquid detergent as sole carbon source were investigated.  Serial dilutions of the  water samples were obtained and 0.1ml of the dilutions was spread plated in duplicates onto the detergent agar medium. The plates were then incubated for 72hrs at room temperature. Organisms growing on the plates were counted, sub-cultured and stored for identification,

Characterization and identification of detergent-utilizing isolates

Bacterial isolates were characterized and identified based on cultural, morphological and biochemical tests (Gram stain, indole, catalase, voges-proskauer, methyl red, urease, motility, oxidase,  spore stain, citrate utilization and sugar utilization) according to Chesbrough 2006

RESULTS AND DISCUSSION

Table 1 Total Viable Heterotrophic Bacterial Counts

| Sample | Counts (cfu/ml) |
| --- | --- |
| River Niger Water | 1.57 x 106 |
| Tap Water | 3.4 x 105 |

| Table 2: Mean Counts Of Detergent Utilizing Bacteria (cfu/ml) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sample | River water | % | Tap water | % |
| LB wash 'N' wax | 8.2. x 104 | 5.2 | 2.9 x 103 | 0.9 |
| Classic hand wash | 7.7 x 104 | 4.9 | 1.1 x 103 | 0.3 |
| Morning Fresh | 6.4 x 104 | 4.1 | 9 x 102 | 0.2 |
| Mama Lemon | 8.0 x 104 | 5.1 | 1.3 x 103 | 0.4 |
| Newday Fresh | 1.0 x 106 | 63.7 | 7.5 x 103 | 2.2 |
| Lily Fresh SDS | 1.04 x 106 3 x 104 | 66.2 1.9 | 9.3 x 103 5 x102 | 2.7 0.1 |

Percent of Calculaltion= UtilizersHeterotrophic Population x 1001

Table 3: Characteristics of Bacterial Isolates

__________________________________________________________________________

Characteristics		Isolate1	Isolate2	Isolate3	Isolate4	Isolate5 Isolate6	Isolate7	Isolate8

_____________________________________________________________________________________________Morphology		Moist	Smooth		Smooth		Smooth		Smooth

Small	Slightly		Slightly	Small	Small		Shiny

Single	Curved	In Pairs	Curved	Moist	Slender	Straight	Rough

Microscopic morphology	Rods	Rods	Cocci	Rods	Rods	Rods	Rods

Gram reaction		-	-	+	+	-	-	-	-

Motility			-	+	-	+	+	-	-	-

Catalase			+	+	-	+	+	+	+	+

Voges Proskaeur		-	-	-	-	-	-	+	-

Methyl Red		+	-	-	-	-	-	-	+

Citrate			-	-	+			+	-	-

Urease			+	-	-	-	-	-	+	+

Oxidase			-	+	-			+	-	-

Indole			+	-	-	+	-	+	+	+

Spore stain		-	-	-	-	-	-	+	-

Glucose			A	A-	A-	-	AG	A-	-	AG

Fructose			A-	-	-	AG	-	A-	AG	AG

Maltose			AG	-		AG	-	A-	AG	AG

Lactose			AG	AG	A-	AG	AG	A-	AG	AG

Probable Identity		E.coli	Pseudo	Staphy	Bacci	Salmo	Flavo	Entero	Citro

Monas	lococcus	 lus	nella	bacte	bacter	bacter

spp.	spp	 spp.	spp.	rium	spp.	spp.

spp.

_____________________________________________________________________________________________

| Table 4: Changes in pH (Mean) During the River Water Die-Away Test |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (day) | LB wash 'N' wax | Classic hard wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| 0 | 7.20 | 7.50 | 7.30 | 6.80 | 6.70 | 6.75 | 6.52 | 6.30 |
| 4 | 6.80 | 6.90 | 6.70 | 6.95 | 6.85 | 6.80 | 6.69 | 6.35 |
| 8 | 7.30 | 6.60 | 7.10 | 7.20 | 6.93 | 6.85 | 7.24 | 6.42 |
| 12 | 6.6 | 6.40 | 7.40 | 7.53 | 7.25 | 6.92 | 7.76 | 6.48 |
| 16 | 7.50 | 6.20 | 7.80 | 7.90 | 7.56 | 7.32 | 7.85 | 6.53 |
| 20 | 7.60 | 6.70 | 8.20 | 8.32 | 7.73 | 7.65 | 8.22 | 6.60 |
| 24 | 7.65 | 6.80 | 8.45 | 8.50 | 7.95 | 7.98 | 8.35 | 6.62 |
| 28 | 7.72 | 7.60 | 8.60 | 8.64 | 8.70 | 8.25 | 8.50 | 6.65 |

Table 5: Changes in Total Viable Counts (Mean) during the River Water Die-Away Test       (Cfu/ml)

| Time (day) | LB wash 'N' wax | Classic hard wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1.0 x 104 | 1.3 x 104 | 1.2 x 104 | 1.3x104 | 1.5. x 104 | 1.6 x 104 | 1.4 x 104 | 2.0 x 104 |
| 4 | 4.1. x 104 | 6.2 x 104 | 7.1. x 104 | 5.8 x 104 | 8.6 x 104 | 9.2 x 104 | 3.0 x 104 | 2.5 x 104 |
| 8 | 5.3 x 104 | 7.0 x 104 | 9.2 x 104 | 7.5 x 104 | 9.0 x 104 | 1.01 x 105 | 4.5 x 104 | 3.6 x 104 |
| 12 | 6.2 x 104 | 7.9 x 104 | 9.7 x 104 | 8.0 x 104 | 9.7 x 104 | 1.28 x 105 | 6.2 x 104 | 4.5 x 104 |
| 16 | 9.2 x 104 | 8.8 x 104 | 1.0 x 105 | 9.0 x 104 | 1.08 x 105 | 1.42 x 105 | 8.0 x 104 | 4.0 x 104 |
| 20 | 9.4 x 104 | 9.0 x 104 | 1.13 x 105 | 9.8 x 104 | 1.33 x 105 | 1.65 x 105 | 8.7 x 104 | 3.0 x 104 |
| 24 | 8.0 x 104 | 7.7 x 104 | 9.9 x 104 | 8.5 x 104 | 1.01 x 105 | 1.48 x 105 | 7.8 x 104 | 3.0 x 104 |
| 28 | 7.6 x 104 | 7.1 x 104 | 9.4 x 104 | 7.8 x 104 | 9.8 x 104 | 1.26 x 105 | 6.0 x 104 | 2.7 x 104 |

| Table 6 : Chemical oxygen demand, Inorganic sulphate and Nitrate concentrations of the original liquid detergents (before addition into River Water) |  |  |  |
| --- | --- | --- | --- |
| Sample | COD (mg/L) | SO42- (mg/L) | NO3-(mg/L) |
| LB wash 'N' wax | 1650 | 750 | 300 |
| Classic hand wash | 2150 | 960 | 200 |
| Morning Fresh | 1560 | 650 | 250 |
| Mama Lemon | 1920 | 840 | 150 |
| New day Fresh | 3250 | 1250 | 350 |
| Lily Fresh | 3900 | 1500 | 300 |
| Sodium dodecyl sulphate (SDS) | 2000 | 800 | 250 |

| Table 7: Frequencies of Isolation of the Detergent -Utilizing Bacteria |  |  |  |
| --- | --- | --- | --- |
| Genus | Number of positive samples | Frequency of Isolation (%) |  |
| Pseudomonas spp | 4 | 28.57 |  |
| Staphylococcus spp | 1 | 7.14 |  |
| Escherichia spp | 2 | 14.29 |  |
| Salmonella spp. | 1 | 7.14 |  |
| Flavobacterium spp. | 2 | 14.29 |  |
| Enterobacter spp. | 1 | 7.14 |  |
| Bacillus spp. | 2 | 14.29 |  |
| Citrobacter spp. | 1 | 7.14 |  |

Table 8: Bacterial Isolates and the detergents utilized

| Genus | Detergents utilized |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LB | CW | MF | ML | NF | LF | SDS |
| Pseudomonas spp. | - | + | - | - | + | + | + |
| Staphylococcus spp | - | - | - | - | - | + | - |
| Escherichia spp | - | - | - | - | + | + | _ |
| Salmonella spp | - | - | - | - | - | + | - |
| Flavobacterium spp | - | - | - | - | - | + | + |
| Enterobacter spp | - | - | - | - | - | + | - |
| Bacillus spp | + | - | + | - | - | - | - |
| Citrobacter spp | - | - | - | + | - | - | - |

Key: +  = Utilized,   -   = not utilized

Table 9: Changes in COD Level during the River Water Die-Away Test

| Time (days) | LB wash 'N' wax | Classic hand wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 165 | 200 | 140 | 150 | 200 | 250 | 200 | 100 |
| 4 | 160 | 170 | 125 | 130 | 180 | 220 | 170 | 80 |
| 8 | 140 | 155 | 115 | 110 | 160 | 190 | 170 | 70 |
| 12 | 120 | 140 | 100 | 90 | 125 | 170 | 130 | 60 |
| 16 | 90 | 120 | 80 | 90 | 100 | 150 | 85 | 50 |
| 20 | 70 | 100 | 60 | 90 | 70 | 110 | 60 | 25 |
| 24 | 50 | 80 | 50 | 70 | 40 | 70 | 45 | 25 |
| 28 | 25 | 30 | 35 | 45 | 30 | 45 | 35 | 20 |

Table 10: Changes in SO42- Concentration during the River Water Die-away Test

| Time (days) | LB wash 'N' wax | Classic hand wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 75 | 95 | 65 | 84 | 120 | 150 | 80 | 45 |
| 4 | 65 | 80 | 50 | 75 | 100 | 115 | 70 | 40 |
| 8 | 50 | 70 | 40 | 60 | 80 | 90.50 | 60 | 33 |
| 12 | 35 | 55 | 30.50 | 50 | 65 | 75 | 50.50 | 30 |
| 16 | 25 | 40 | 25 | 40 | 50.50 | 60 | 40 | 20 |
| 20 | 20 | 25 | 20 | 23 | 35.50 | 40 | 30 | 15 |
| 24 | 10 | 15 | 10 | 14.50 | 20 | 35 | 20 | 12 |
| 28 | 5 | 8 | 6 | 7.50 | 10 | 10.50 | 9.50 | 10 |

Table 11: Changes in NO3- Concentration during the River Water Die-away Test

| Time (days) | LB wash 'N' wax | Classic hand wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 30.00 | 20.30 | 25.00 | 15.00 | 30.00 | 30.35 | 20.50 | 10.00 |
| 4 | 24.35 | 16.47 | 20.56 | 12.50 | 25.20 | 25.00 | 15.00 | 8.50 |
| 8 | 20.00 | 11.00 | 22.00 | 10.00 | 18.15 | 20.14 | 10.31 | 7.00 |
| 12 | 15.22 | 7.84 | 13.86 | 7.32 | 11.83 | 13.52 | 7.50 | 6.00 |
| 16 | 8.63 | 6.51 | 9.01 | 5.64 | 9.20 | 8.90 | 6.00 | 5.50 |
| 20 | 5.84 | 5.03 | 6.72 | 4.08 | 6.21 | 6.37 | 4.80 | 4.10 |
| 24 | 3.55 | 3.00 | 3.60 | 3.00 | 4.00 | 3.22 | 2.94 | 3.25 |
| 28 | 3.21 | 2.62 | 1.85 | 2.13 | 3.05 | 2.45 | 2.40 | 1.80 |

Table 12: Changes in % Surfactant remaining during the River Water Die-away Test

| Time (days) | LB wash 'N' wax | Classic hand wash | Morning fresh | Mama lemon | New day fresh | Lily fresh | SDS | Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 4 | 90.31 | 94.20 | 96.50 | 97.63 | 93.25 | 95.00 | 92.50 | 100 |
| 8 | 85.00 | 90.50 | 91.12 | 93.45 | 89.58 | 90.25 | 85.00 | 100 |
| 12 | 65.00 | 70.00 | 83.34 | 90.29 | 73.60 | 80.00 | 70.50 | 50.00 |
| 16 | 58.25 | 63.57 | 70.69 | 80.82 | 50.54 | 55.40 | 45.50 | 50.00 |
| 20 | 52.50 | 57.00 | 60.50 | 65.00 | 35.00 | 36.52 | 25.00 | 50.00 |
| 24 | 45.35 | 52.85 | 55.20 | 51.75 | 28.30 | 25.63 | 15.75 | 0 |
| 28 | 40.00 | 49.25 | 50.00 | 43.00 | 22.50 | 15.00 | 10.00 | 0 |

The research showed that the liquid detergents supported appreciable growth of microorganisms.  The amount of microbial numbers growing on the detergent agar media may be indicative of either the ease of biodegradation of the detergent components or the efficacy of the biocides incorporated in the detergent formulations rather than bioavailability of the surfactant components. This supported the observations of Okpokwasili and Mbipom (2004).

The mean total viable counts of the cultures increased from the initial day to the 20th day before decreasing during the 28- day monitoring period. The mean total viable counts increased fastest with Lily Fresh and Newday Fresh and least with Classic Handwash and Sodium Dodecyl sulphate. The production of waste products (some of which may be toxic) with time which accumulated in the medium, and the gradual depletion of nutrients in the medium due to the metabolic activities of microorganisms could account for the general decrease in bacterial counts after the 20th day.

The increase in pH of the control flask (6.30-6.65) was very gradual compared to the other flasks. This may be due to the fact that it did not contain any detergent and so there were fewer activities by microorganisms present in this flask. However, other samples which had slightly alkaline pH may be due to their chemical compositions.

All the samples contained sulphates and nitrates including the control (river water), although there was a gradual decrease in the COD. Sulphates and nitrates infers that the microbial consortium utilized some of the components during their metabolic activities. High concentrations of nitrates in drinking water can result in a condition known as blue baby syndrome in infants. Nitrate is very soluble in water and can move readily through soil to the groundwater.(USEPA, 2010).

The increase in cell numbers as the natural microbial communities in the river water samples utilized the detergents may be due to the utilization of the surfactants as sole carbon and energy sources for growth and metabolic processes. The overall increase in bacterial numbers in the river water/ detergent cultures may be attributed to the availability of carbon source for growth as well as sulphates concentration in the various detergents. Lily Fresh and Newday Fresh with high sulphate concentrations exhibited higher percentage degradation while Morning Fresh with the least sulphate concentration recorded the least percentage degradation. Lily Fresh and Newday Fresh with high chemical oxygen demand also exhibited higher percentage degradation while Morning fresh had the least in both COD and percentage degradation. The difference in the rate of biodegradation may be due to variations in the concentrations of methylene blue active substances used by different industries in the manufacture of different brands of detergent. The relationship between surfactants and microbes is complex and involve factors other than biodegradation and under appropriate conditions, surfactants can act as bactericides and bacteriostats (Smulders, 2009). However, the ability of a surfactant to be bactericidal depends largely on the microbial species, size of the hydrophobic portion of the surfactant molecule, purity of the water sample in terms of organic matter such as sewage and the presence of divalent metal ions (Rosen, 2010).

One important factor that affects surfactants degradation is the structure of the surfactants. Schleheck et al (2004) showed that the hydrophobic group is a very important determinant of biodegradability. Increased distance between the sulphonate and the far end of the hydrophobic group changes the speed of the biodegradation of alkylbenzenesulphonates (ABS). The degree of branching also affects the degradative ability of a given surfactant (Scott, 2000).

Lily Fresh and Newday Fresh which were readily utilized may contain compounds that are directly utilized by those microorganisms containing the appropriate enzymes, which are able to catalyze these reactions. Morning Fresh and SDS that were not readily utilized may contain components that are recalcitrant or the available microorganisms (bacteria) are not capable of developing matching enzymes to carry out the degradation, which may determine a surfactant’s recalcitrance or biodegradability.

The bacteria found growing in the liquid detergents were mainly Gram- negative rods. This suggests that they are more tolerant to the surfactant concentrations present in the isolation medium than the Gram- positive bacteria (Smulders, 2007) have reported that many Gram- positive bacteria are affected by surfactant concentrations of 10-20 parts per million (ppm) while several thousand ppm may be without effect on Gram- negative organisms. Some of the bacterial genera found capable of utilizing the liquid detergents have been reported as capable of assimilating portions of pure anionic surfactant molecules.

Statistical analysis results showed the there is a relationship between the pH values and total viable counts of the samples. There is relationship between the COD values and total viable counts of the samples; a relationship between the sulphate concentrations and total viable counts of the samples and lastly, a relationship between the nitrate concentrations and total viable counts of the samples. These imply that the variations in the parameters-pH, COD, sulphate and nitrate values resulted in an increase in the total viable counts of the samples during the biodegradation test. (Scott, 2000).

References.

APHA, (2002). Standard Methods for the examination of water and wastewater. American Public Health Association. Baltimore Maryland, U.S.A. pp.1, 325.

Banat, I.M. (2000). Potential for use of biosurfactants in oil spills clean up and oil bioremediation. Proceedings of Oil Hydrocarbon Spills 11, Modeling, Analysis and Control. (Rodriguez, G.R. and Brebbia, C.A. (eds)). WIT Press, Southampton. pp. 177-185.

Holt, J.G., Krieg, N.R., Sneath, P.H.A., Staley, J.T. and Williams, S.T. (1994). Bergey’s Mannual of Determinative Bacteriology. 9th edition. Williams and Wilkins Company, London.

Kosswig, K. (2009). Surfactants. In Ullmann’s Encyclopedia of Industrial Chemistry. (Bohnet, M., Brinker, C.J., Clemens, H. and Cornils, B. (eds.)). Wiley-VCH, Weinheim, Germany.

Kuritz, T. and Wolk, C.P. (1995). Use of Filamentous cyanobacteria for biodegradation of organic pollutants. Applied Environmental Microbiology 61: 234-238.

Luders, H. (2000). Chemical Properties and Derivation of surface active Alkyl Glycosides and Alkyl Polyglucosides Science. In Non ionic Surfactants/Alkyl Polyglucosides, Surfactants Series. Marcel Dekker Ltd., New York. 91: 77-83.

Madsen, T., Boyd, H.B., Nylen, D., Petersen, A., Petersen, G.I. and Flemming, S. (2001). Environmental and health assessment of substances in household detergents and cosmetic detergent products. Environmental Project No. 615. CETOX.

Maguire, R.J. (1999). Review of the persistence of nonylphenol and nonylphenol ethoxylates in aquatic environments. Water Quality Resource Journal of Canada. 34(1): 37-78.

Meade, R. (1995). Organic Contamination of the Mississippi River from Municipal and industrial waste water. U.S Geological Survey Circular 1133. Reston, Virginia. pp. 1-18.

Merrettig –Bruns, U. and Jelen, E. (2009). Anaerobic Biodegradation of Detergent Surfactants. Materials. 2: 181-206.

Metcalfe, T.L., Dillon, P.J. and Metcalf, C.O. (2008). Detecting the transport of toxic pesticides from Golf courses into watersheds in the precambian shield region of Ontario, Canada. Environmental Toxicology and Chemistry. 27(4): 811-818.

Murphy, M., Alkhalidi, M., Crocker, J., Lee, S., Oregan, P. and Acott, P. (2005). Two formulations of the industrial surfactant, Toximul, differentially reduce mouse weight gain and hepatic glycogen invivo during early development: effects of exposure to Influenza B virus. Chemosphere. 59(2): 235-246.

Nomura, Y., Ikebukuro, K., Yokoyama, K., Takenchi, T., Arikawa, Y., Ohno, S. and Karube, I. (1998). Application of a linear alkylbenzene sulfonate biosensor to river water monitoring. Biosensor Bioelectric 13: 1047-1053.

Ogu, T.C., Okaa,  A.I.,Ozokpo, A.C. and Onochie, C.C.(2016).  Microbial deterioration of paints. African  journal of Education , Science and Technology, 3(1):190-194.

Ohalete, C.L., Umeh, S.O. and Nwajiobi,F.O. (2024). Isolation and Screening of Trichosporon Asahii for biosurfactant production and optimization. International Journal of Research and Innovation in Applied Sciences, 9(4): 413-424. DOI: https//doi.org/10.51584.

Ojo, O.A. and Oso, B.A. (2008). Isolation and characterization of synthetic detergent degraders from wastewater. African Journal of Biotechnology. 7(20): 3753-3760.

Okpokwasili, G.C. and Mbipom, R.E. (2004). Primary Degradation of Surfactants in Liquid Detergents. Nigerian Journal of Microbiology. 18(1-2): 387-391.

Okpokwasili, G.C. and Okorie, B.B. (1991). Influence of physicochemical stress on biodegradability of car engine lubricating oil. International Biodeterioration. 27: 255-264.

Okpokwasili, G.C. and Olisa, A.O. (1991). River Water Biodegradation of Surfactants in Liquid detergents and shampoos. Water Resource 25(11): 1425-1429.

Oyeka, C.A. (1996).  An Introduction to applied Statistical Methods. 7th edition. Modern Printers, Eneme-Enugu. pp. 206, 33-334.

Peterssen, G., Rasmussen, D., Maenpaa, K., Kallqvist, T., Madsen, T. and Kukkonem, J.V.K. (2003). Transport and fate of surfactants in the aquatic environment. Technical report 524. NORDTEST, Finland.

Prats, D., Carmen, L., Diana, V., Pedro, V. and Victor, M.L. (2006). Effect of Temperature on the Biodegradation of Linear Alkyl benzene sulfonate and Alcohol ethoxylate. Journal of Surfactants and Detergents. 9(1):69-75.

Prosksova, M., Sladekova, D., Uhnakova, B., Augustin, J. and Toth, D. (1999). Biologia, Bratislava. 54: 671-677.

Radosevich, M., Jin, Y. and Cha, D. (1999). The effect of Biosurfactant on the fate and transport of non-polar organic contaminants in porous media. Department of Plant and Soil Sciences, University of Delaware, New York. pp. 1-34.

Rahman, K.S.M., Thahira-Rahman, J., Mcclean, S., Marchant, R. and Banat, I.M. (2002). Rhamnolipid biosurfactants production by strains of Pseudomonas aeruginosa using low cost raw materials. Biotechnology Programme 18: 1277-1281

Ren, X. (1998). Application of Biosurfactant in Naphthalene, Phenanthrene and TCE Remediation. Ph.D Dissertation. University of Houston, Houston, Texas.

Reznik, G.O., Vishwanath, P., Pynn, M.A., Sitnik, J.M., Todd, J.J., Wu, J., Jiang, Y. and Keenan, B.G. (2010). Use of sustainable chemistry to produce an acyl amino acid surfactant. Applied Microbiology and Biotechnology. 86(5): 1387-1397.

Rosen, M.J. (2010). Surfactants and Interfacial Phenomena. 3rd Edition. John Wiley and Sons. Hoboken, New Jersey. p. 1.

Ross, M. (2000). The World Book Encyclopedia. Vol.5. Scott Fretzer. Chicago, U.S.A. pp.163-166.

Schleheck, D., Knepperr, T.P., Fischer, K. and Cook, A.M. (2004). Mineralization of individual congeners of linear alkylbenzene sulfonate by defined pairs of heterotrophic bacteria. Applied Environmental Microbiology 70(7) : 4053-4063.

Schwatz, A. and Bar, R. (1995). Cyclodextrin enhanced degradation of toluene and toluic acid by Pseudomonas putida. Applied Environmental Microbiology. 61: 2727-2731.

Scott, M. (2000). The biodegradation of Surfactants in the environment. Biomembranes. 1508: 235-251.

Smulders, E., Von-Rybinski, W., Sung, E., Rahse, W., Steber, J., Wiebel, F. and Nordskog, A. (2002). Laundry Detergents. In Ullmann’s Encyclopedia of Industrial Chemistry. Wiley-VCH, Weinheim, Germany.

Smulders, E., Von-Rybinski, W., Sung, E., Rahse, W., Steber, J., Wiebel, F. and Nordskog, A. (2007). Laundry Detergents. In Ullmann’s Encyclopedia of Industrial Chemistry. Wiley-VCH, Weinheim, Germany.

Smulders, E., Von-Rybinski, W., Sung, E., Rahse, W., Steber, J., Wiebel, F. and Nordskog, A. (2009). Laundry Detergents. In Ullmann’s Encyclopedia of Industrial Chemistry. (Bohnet, M., Brinker, C.J., Clemens, H., Cornils, B., Evans, T.J., Greim, H., Hegedus, L.L., Heitbaum, T., Herrmann, W.A. and Karst, U. (eds.)). Wiley-VCH, Weinheim, Germany.

United States Environmental Protection Agency (USEPA) (2010). Eco-regional nutrient criteria as water quality standards. Office of Health and Environmental Assessment, US Environmental Protection Agency. Washington, D.C.